Ames Research Center

Moffett Field, California

Lunar Atmosphere and Dust Environment Explorer

(LADEE)

(NMS PDS Software Interface Specification)

Mehdi Benna

(04/24/2013)

National Aeronautics and

Space Administration

National Aeronautics and

Space Administration

This document is approved in accordance with LADEE Configuration Management Plan, C04.LADEE.CM, paragraph 3.6.1.1 Document Release Routing Approval Process.

Page three of this document contains  the approved routed release of this document.

Approval Signatures

_______________________________________		___________

Dr. Mehdi Benna				     		Date

Instrument Scientist, NMS

_______________________________________		___________

Dr. Paul Mahaffy			     			Date

Principal Investigator, NMS

_______________________________________		___________

Dr. Greg Delory				     		Date

LADEE Deputy Project Scientist

_______________________________________		___________

Dr. Reta Beebe				     		Date

PDS Atmospheres Node Manager

_______________________________________		___________

Dr. Tom Morgan					     	Date

Project Manager, PDS

This page reserved for routing approval report

| REVISION HISTORY |  |  |  |
| --- | --- | --- | --- |
| Rev. | Description of Change | Author(s) | Effective Date |
| Draft A | Initial Draft | Mehdi Benna | 04/17/2013 |
| Rev 1.0 | First Release | Mehdi Benna | 04/24/2013 |
| Rev 1.1 | Update of section 3.2.1 | Mehdi Benna | 04/30/2013 |

CONFIGURATION MANAGEMENT PLAN

This document is an LADEE Project Configuration Management (CM)-controlled document.  Changes to this document require prior approval of the LADEE Project Manager.  Proposed changes shall be submitted to the LADEE CM office along with supportive material justifying the proposed change.  Changes to this document will be made by complete revision.

Questions or comments concerning this document should be addressed to:

LADEE Project Control Manager

Mail Stop 240-5

Ames Research Center

Moffett Field, California 94035

ACRONYMS	viii

1.	INTRODUCTION	9

1.1	Purpose and Scope	9

1.2	Contents	9

1.3	Applicable Documents and Constraints	9

1.4	Relationships with Other Interfaces	9

2.	MANAGEMENT AND OVERSIGHT	10

3.	DATA PRODUCT CHARACTERISTICS AND ENVIRONMENT	10

3.1	Instrument Overview	10

3.1.1	Ion Sources	11

3.1.2	Ion Optics	13

3.1.3	Mass Analyzer	13

3.1.4	Detectors	14

3.1.5	Instrument Accommodation on the LADEE spacecraft	14

3.1.6	Instrument Calibration	15

3.2	Data Products	15

3.2.1	NMS Product Definitions	15

3.2.2	Data Product Detailed Description and Format	15

3.2.3	Data Products Generation	16

3.2.4	Data Validation	16

4.	ARCHIVE VOLUMES	16

4.1	Generation	16

4.2	Data Transfer	16

4.3	Review and Revision	17

4.4	Data Volume Architecture	17

5.	ARCHIVE RELEASE SCHEDULE	19

6.	COGNIZANT PERSONS	20

7.	REFERENCES	20

8.	APPENDICES	21

8.1	NMS Electrodes Designation	21

8.2	NMS DAC ID designation:	22

8.3	NMS Data Product Column Descriptions	24

8.3.1	Raw housekeeping data table	24

8.3.2	Raw science data table	38

8.3.3	Raw message log	38

8.3.4	Calibrated housekeeping table	38

8.3.5	Calibrated science data table	52

8.3.6	Time-corrected message log	52

# ACRONYMS

| ATM C&DH DAC DAWG DMAP EOM GSFC ICD I&T INMS LADEE LLCD NMS NGIMS OLAF PDS RF SIS SQL SOC TBD | Planetary Atmospheres Node Command and Data Handling Digital to Analogue Converter Data and Archives Working Group Data Management and Archive Plan End of (LADEE) Mission Goddard Space Flight Center Interface Control Document Integration and Testing Ion and Neutral Gas Mass Spectrometer Lunar Atmosphere and Dust Environment Explorer Lunar Laser Communication Demonstrator Neutral Mass Spectrometer Neutral Gas and Ion Mass Spectrometer Online Archive Facility Planetary Data System Radio Frequency Software Interface Specification Structured Query Language Science Operation Center To Be Determined |
| --- | --- |

# INTRODUCTION

# Purpose and Scope

This document describes the format and the content of the Neutral Mass Spectrometer (NMS) products as archived in the Planetary Atmospheres Discipline Node (ATM) in PDS. The data products stored in PDS are a subset of the holdings of the NMS team database at NASA’s Goddard Space Flight Center (GSFC).

This SIS is intended to provide enough information to enable users to read and understand the NMS data products as stored in PDS. The users for whom this SIS is intended are software developers of the programs used in generating the NMS products and scientists who will analyze the data, including those associated with the LADEE mission and those in the general lunar science community.

# Contents

NMS is an instrument on the LADEE spacecraft designed to analyze the composition of the lunar exosphere during the mission. This Data Product SIS describes how the NMS instrument acquires its data and how the data are processed.

# Applicable Documents and Constraints

- Planetary Data System Standards Reference, JPL D-7669 part 2, version 4.0.6, October 8, 2012.

- LADEE Project Data Management and Archive Plan, version 2.2, May, 2011.

- LADEE NMS PDS Interface Control Document, version 1.1 April 24, 2013.

- LADEE NMS Calibration Report, version 0, April 1, 2013.

# Relationships with Other Interfaces

Figure 1: The LADEE NMS Instrument during Integration and Testing (I&T).

The NMS data products are stored in multiple locations according to the LADEE Project Data Management and Archive Plan (DMAP). The master copy stored in an SQL (Structured Query Language) relational database for rapid instrument team access will be used by the NMS science team to retrieve and process data for delivery to PDS as described by the LADEE NMS PDS Interface Control Document.

# MANAGEMENT AND OVERSIGHT

Data will be produced by the NMS science team for submission to PDS.  Data delivered to PDS will be managed and verified according to the LADEE NMS PDS Interface Control Document and the PDS Standards Reference.

# DATA PRODUCT CHARACTERISTICS AND ENVIRONMENT

# Instrument Overview

Neutral gas and ions

Collimators

Open source ionization region

Antechamber

Closed source ionization region

Quadrupole deflectors

Ion lens system

Quadrupole mass filter

Ion detector

Secondary ions

Figure 2: Schematics illustrating the principal components of the NMS sensor.

The LADEE Neutral Mass Spectrometer (NMS) is a high sensitivity quadrupole mass spectrometer with a mass range of 2 to 150 Dalton and unit mass resolution (Figure 1). The sensor of the NMS instrument is the upgraded engineering unit of the Neutral Gas and Ion Mass Spectrometer (NGIMS) developed for the CONTOUR mission [1]. This mass spectrometer is similar to the CASSINI Ion and Neutral Gas Mass Spectrometer (INMS) designed and developed at GSFC [2].

The LADEE NMS instrument was modified from the heritage CONTOUR NGIMS instrument to increase the instrument sensitivity, field of view and overall operational flexibility.

The NMS sensor consists of:

- two separate ion sources for sampling ambient neutrals and ions,

- an ion deflector/trap,

- four hot-filament electron guns,

- an electrostatic quadrupole switching lens that selects between the sources,

- various focusing lenses,

- a quadrupole mass analyzer, and

- two secondary electron multiplier (SEM) detectors.

The instrument control is provided by the Command and Data Handling (C&DH) unit, according to the instructions given to a user defined script. The C&DH and all the related electronics boards are packaged together. A sketch of the key NMS components is shown in Figure 2, and the primary instrument parameters are listed in Table 1.

# Ion Sources

The NMS instrument uses two separate ion sources, a closed source and an open source in order to optimize interpretation of the neutral species (Figure 3).

In the closed source mode, the ram pressure of the inflowing gas creates a density enhancement in the source antechamber, allowing the sampled species to be measured with relatively high precision and sensitivity. This mode will be used to measure species, such as He and Ar, which do not react with the antechamber surfaces.

Table 1: Key NMS parameters

| NMS Instrument Parameters |  |
| --- | --- |
| Neutral gas sampling systems | - Closed source (non wall reactive species) - Open source (wall reactive species) |
| Ion sampling system | Thermal and suprathermal positive ions |
| Source switching system | Electrostatic quadrupole deflector |
| Field of view | - Closed source: 2 steradians - Open source 10 cone half angle |
| Neutral mode ionization sources | Electron impact ionization with redundant filaments - Closed source: 200 A and 70eV - Open source: 200 A and 70eV |
| Mass analyzer | Quadrupole mass filter; 0.508 cm field radius, 15 cm rod length; Radio frequencies: 1.43 and 3.10 MHz |
| Mass range | 2 – 150 Daltons |
| Scan modes | - Survey: scan mass range in 0.1 Daltons steps - Adaptive mode: select mass values |
| Crosstalk | 10-6 for adjacent masses |
| Detector system | Two secondary electron multiplier detectors operating in pulse counting mode (detector noise <1 count per min) Dynamic range ~ 108 |
| Data rate | Integration period from 27 ms to 250 ms with a 3 ms setup time per period. |

# 75

# 1

# 2

# 3

# 4

# 5

# 6

# 7

# 8

# 18

# 25

# 22

# 26

# 20

# 19

# 23

# 24

# 21

# 9

# 14

# 11

# 12

# 13

# 10

# 15

# 16

# 17

# 41

# 38

# 34

# 36

# 35

# 40

# 37

# 33

# 42

# 43

# 39

# 46

# 44

# 45

# 47

# 50

# 48

# 31

# 32

# 27

# 28

# 29

# 30

# 51

# 52

# 54

# 53

# 56

# 55

# 57

# 58

# 59

# 60

# 61

# 62

# 63

# 68

# 64

# 66

# 67

# 65

# 69

# 70

# 71

# 62

# 73

# 74

# 49

Open Source

Closed Source

Antechamber

Transfer tube

Electron gun # 1

Electron gun # 2

Switching Lenses

Ion Focusing

Quadrupole Mass Filter

Detector

Electron gun # 1

Electron gun # 2

Neutrals and ions

Neutrals and ions

Ions path

Electron beam (primary gun)

Electron beam (secondary gun)

Figure 3: Schematics of NMS electrostatic elements. Table 3 provides the related nomenclature.

The open source has the advantage that it can measure reactive neutral radicals, such as atomic oxygen, and ions. In this mode, the ambient neutral gas density is sampled directly with no stagnation enhancement and no collisions with the surfaces of the instrument. For open source ion measurements, the NMS angular response can be increased beyond the geometric view cone by adjusting the voltages on the ion collimator lenses. For neutral sampling in the open source mode, the ion collimator lenses and the repeller lens remove incoming ions and electrons, which could cause spurious ionization of neutral species, and allow only neutrals to pass into the ionization region.

# Ion Optics

In both closed and open source modes, impacting electrons emitted from the hot-filament electron guns ionize the sampled neutrals. Electrostatic lenses are used to focus the ambient ions and those created from ambient neutrals by electron impact into the quadrupole switching lens (Mahaffy and Lai, 1990), an electrostatic device that steers ions from either the closed or open source through a system of focusing lenses into a dual radio frequency (RF) quadrupole mass analyzer.

# Mass Analyzer

The mass analyzer selectively filters the ions according to their mass-to-charge ratio using a set of 4 hyperbolic rods excited with a RF wave form.

Two opposing potentials of the form UDC+UACcos2πft drive each pair of rods (Figure 4). Ions with the appropriate ratio of mass to charge achieve stable trajectories while the rest of the ions diverge and end up impacting the rods.

+(UDC+UACcos2πft)

+(UDC+UACcos2πft)

Figure 4: Quadrupole mass filter and driving RF potentials.

During a mass scan the absolute values of UDC and UAC are increased while the ratio UDC/UAC is kept constant. The DC and AC potentials are calculated for the given target mass as:

UDC= m r02 π2 f2 a2 e

UAC= m r02 π2 f2 qe

Where m is the mass of the targeted ion, r0 is the hyperbolic rods radius, f the RF frequency, and  e the electron charge.  a ≈0.23699 and q ≈0.7060 are constants that drive the mass resolution of the analyzer.

In order to cover the mass range of 2 to 150 Da while keeping the voltages relatively low, the RF frequency is switched from f1=1.43 MHz to f2=3.10 MHz at mass m=21 Da.

When the NMS is operating in the open source mode, a quadrupole bias voltage UQB is added to the DC voltage applied to the RF mass analyzer rods to slow down incoming ions and increase their residence time in the analyzer’s RF field.

# Detectors

Ions exiting the quadrupole mass filter are detected by one of two redundant secondary electron multipliers. The multipliers are associated electrodes electronically biased such that most of the ions are deflected into one of the detectors. Charge pulses at the anode of the multiplier are amplified and counted. The detection threshold is determined by the background noise in the multiplier (approximately one count per minute). The upper count rate of each detector system is about 10 MHz, limited by the product of the multiplier pulse width and gain bandwidth of the pulse amplifier counter system. There is a non-linear response that occurs in the range of 1–10 MHz and needs to be accounted for.

# Instrument Accommodation on the LADEE spacecraft

NMS

Interface plate

Apertures

Figure 5: NMS accommodation on the spacecraft.

Payload module

The NMS instrument is mounted on the Payload Module on the opposite panel from the Lunar Laser Communication Demonstrator (LLCD). The outward normal to both the open and closed source INMS apertures lies in the spacecraft −X direction. The instrument apertures are directed toward the nominal spacecraft motion direction with a clear field of view of any spacecraft structure in order to avoid any induced contamination (from spacecraft structure or thrusters firing).

# Instrument Calibration

NMS was designed, built and tested at the Planetary Environment Laboratory (Code 699) of NASA’s Goddard Space Flight Center (GSFC). During I&T, the NMS instrument was mounted on a vacuum chamber in order to characterize its sensitivity for a set of gases and gas mixes. Detailed descriptions of the calibration procedure and calibration results are provided in the NMS calibration report.

# Data Products

This document uses the LADEE data definitions for all products.  These data have been reviewed and accepted by PDS to comply with anticipated PDS4 standards.  NMS will deliver both raw and calibrated data to PDS as defined in the LADEE definitions table, Table 2, and delineated in Table 3.

# NMS Product Definitions

All NMS products delivered to the PDS are in “spreadsheet” format with comma-delimited columns or as ASCII text files. These products are described in Table 3. Deliveries will be made to PDS in accordance with the schedule defined in the LADEE NMS PDS Interface Control Document.

# Data Product Detailed Description and Format

Data generated by the NMS instrument will be organized in products according to their processing state. NMS data processing pipeline adheres to the nomenclature of product definition set by the LADEE project (Table 2).

Table 2: LADEE Data Processing Levels

| Product | Product Description |
| --- | --- |
| Packet Data | Telemetry data stream as received at the ground station, with science and engineering data embedded. |
| Raw | Original data from an instrument. If compression, reformatting, packetization or other translation has been applied to facilitate data transmission or storage, those processes will be reversed so that the archived data are in a PDS approved archive format. |
| Reduced | Data that have been processed beyond the raw stage but which are not yet entirely independent of the instrument. |
| Calibrated | Data converted to physical units entirely independent of the instrument. |
| Derived | Results that have been distilled from one or more calibrated data products (for example, maps, gravity or magnetic fields, or ring particle size distributions). Supplementary data, such as calibration tables or tables of viewing geometry, used to interpret observational data should also be classified as derived data if not easily matched to one of the other three categories |

The NMS pipeline processes the Packet Data (binary files as generated by the instrument) to generate the Raw, Calibrated and Derived products.  Only these latter products will be archived at the PDS (Table 3).

The Packet Data will be separated by telemetry channel (Housekeeping, Science and Instrument Log) and converted to ASCII to generate the Raw Housekeeping, the Raw Science and the Message Log. These data will be checked for anomalies and will be time-stamped. The Housekeeping units will be expressed in engineering units (the units of the ADC or DAC units) when applicable.

Then, the Raw Science will be corrected for detector response (dead time correction) and the Raw Housekeeping will be converted to scientific units (physical unit corresponding to the measurement being made: deg C for Temp, A for current or emission, and V for voltage monitor circuits) when applicable. These data will be checked for anomalies and the time-stamp will be corrected for any offset between the instrument and spacecraft clocks. This process will yield Calibrated Housekeeping and Calibrated Science ASCII files.

Finally, for a subset of Calibrated data sets a Derived product will be generated to reflect the abundances of key species that exhibit a sufficient SNR. This conversion will assume lab measured sensitivities for He, Ar and Ne and for interpolated sensitivities for the rest of the reported species.

Table 3: Data Definitions for NMS.

| Product Name | Description | Estimated File (B = Bytes) | Type |
| --- | --- | --- | --- |
| Raw Housekeeping | Instrument housekeeping packets | TBD | Raw |
| Raw Science | Instrument science packets | TBD | Raw |
| Message Log | Instrument message log | TBD | Raw |
| Calibrated Housekeeping | Instrument housekeeping packets | TBD | Calibrated |
| Calibrated Science | Instrument science packets | TBD | Calibrated |
| Species Abundance | Abundances of key exospheric species | TBD | Derived |

Appendix A provides detailed information about the formats and content descriptions of the archived data products.

# Data Products Generation

All data products and associated documentation will be generated by the NMS team. The PDS Planetary Atmospheres Discipline Node will assist in the definition and development of first delivery products and their associated PDS documentation, which will act as templates for subsequent updates. When new products are developed by the NMS team, PDS Planetary Atmospheres Discipline Node will likewise assist in the definition and development of those products and their associated PDS documentation in preparation for their initial delivery.

# Data Validation

Data content validation will be performed by the NMS science team prior to delivery to PDS.  Data structure and format will be performed by the NMS science team and the PDS data review team as described in Section 4.3.

# ARCHIVE VOLUMES

## Generation

The NMS Data Product Archive Collection and its updates are produced by the NMS Instrument Team in cooperation with the Planetary Atmospheres Discipline Node (ATM) of PDS. The Archive Collection will include data acquired during calibration, commissioning, and science phases.

The Planetary Atmospheres Discipline Node and NMS will collaborate to design the PDS documentation files associated with the initial data delivery by the NMS team. All data formats are based on the Planetary Data System standards as documented in the PDS Standards Reference.

## Data Transfer

The NMS team will submit data to PDS via the OnLine Archive Facility (OLAF).  This submission tool is a product of the Planetary Atmospheres Data Node of PDS to help facilitate the exchange of data and will be maintained by PADN.  The NMS and PADN teams will work together in the submission via OLAF to ensure a product meeting the requirements of PDS.

## Review and Revision

The Planetary Atmospheres Discipline Node is responsible for organizing the Peer Review of the NMS data sets, according to PDS policy. The Peer Review Committee will include a small number of scientists, selected by ATM and from outside the NMS Team, who have an interest in the anticipated data products. The Peer Review committee will also include NMS Team members and PDS representatives.

For NMS there will be two such reviews.  There will be a pre-launch review approximately 6 months from launch.  This review will contain sample data and documentation in the format of the final archived data set.  This sample data will be produced by the flight instrument pre-launch and will differ in the final data set only in specific values and size.  Data format and archive method will be the equivalent.

There will be a final review within 6 weeks after the end of mission (EOM).  This review will include all the data produced by NMS from the beginning of the commissioning phase through the first 20 days of the science phase.  Three months after EOM, NMS will deliver the final data set for archive.

## Data Volume Architecture

The complete set of LADEE NMS data will be archived in PDS in a single bundle in the PDS4 standard.  In the outline below, each .csv,  .txt. and .pdf file is assumed to have an .xml label file with the same filename base, which is not mentioned in the outline.  Labels for other types of files are mentioned explicitly.

Root Level of NMS Bundle

Bundle label, including inventory for the bundle (bundle_1.xml)

Bundle table of contents (readme_1.0.txt)

Context Collection – contains mission, spacecraft, instrument, and other context objects.  These context objects refer to the full descriptions in the document collection.

/context

Inventory of context collection (collection_1.0.tab)

Instrument context object (instrument_nms.xml)

Instrument host (spacecraft) context object (instrument_host_ladee.xml)

Investigation (LADEE mission) context object (investigation_ladee.xml)

Target object (target_moon.xml)

Data Collection – contains all data products and their labels

/data

Inventory of the data collection (collection_1.0.tab)

/ground (contains all the ground calibration products)

/raw (contains all the raw products)

Housekeeping data tables (file_name.tab)

Science data table (file_name.tab)

Message log (file_name.txt)

/calibrated (contains all calibrated data products)

Housekeeping data tables (file_name.tab)

Science data table (file_name.tab)

/flight (contains all the flight products)

/raw (contains all the raw products)

Housekeeping data tables (file_name.tab)

Science data table (file_name.tab)

Message log (file_name.txt)

/calibrated (contains all calibrated data products)

Housekeeping data tables (file_name.tab)

Science data table (file_name.tab)

Time-corrected message log (file_name.txt)

/derived (contains all derived products)

Species abundances (file_name.tab)

Document Collection – contains documents relevant to the bundle

/document

Inventory of the document collection (collection_1.0.tab)

NMS instrument calibration report (instrument_calibration_nms.pdf)

LADEE mission description (investigation_ladee.pdf)

LADEE spacecraft description (instrument_host_ladee.pdf)

LADEE NMS bundle description (bundle_nms.txt)

LADEE NMS SIS (instrument_pds_nms.pdf)

Schema Collection – contains the schemas used in the bundle

/xml_schema

Inventory of the schema collection (collection_1.0.tab)

catalog.xml - Lists PDS schemas and namespaces used in the bundle

catalog_label.xml - Label for catalog.xml

PDS4_PDS_0300a.xml - Label for Main PDS schema

PDS4_PDS_0300a.xsd - Main PDS schema – defines the element structure for PDS4

PDS4_PDS_0300a.sch - Main PDS schema – defines valid element values and other semantic constraints for PDS4

# ARCHIVE RELEASE SCHEDULE

Figure 6 shows the delivery schedule in reference to the mission timeline.  Three months from EOM, the final dataset with liens resolved will be delivered.

Figure 6: Data processing timeline against mission events and milestones.

# COGNIZANT PERSONS

Table 4: Cognizant Persons for NMS PDS Data

| NMS Team |  |  |
| --- | --- | --- |
| Principal Investigator, Dr. Paul Mahaffy | Planetary Environment Laboratory NASA Goddard Space Flight Center Code 699 8800 Greenbelt road Greenbelt, MD 20771 | 301 614-6379 Paul.mahaffy@nasa.gov |
| NMS Scientist Dr. Mehdi Benna | Planetary Environment Laboratory NASA Goddard Space Flight Center Code 699 8800 Greenbelt road Greenbelt, MD 20771 | 301 614-6392 mehdi.benna@nasa.gov |
| NMS Scientist Dr. Richard Hodges |  | 301 614-6392 hodges@lasp.colorado.edu |
| NMS Calibration Engineer Mr. Eric Raeen | Planetary Environment Laboratory NASA Goddard Space Flight Center Code 699 8800 Greenbelt road Greenbelt, MD 20771 | 301 614-6363 eric.raeen@nasa.gov |
| NMS Archive Engineer Mr. Matthew Lefavor | Planetary Environment Laboratory NASA Goddard Space Flight Center Code 699 8800 Greenbelt road Greenbelt, MD 20771 | 301 614-6818 Matthew.lefavor@nasa.gov |
| PDS Planetary Atmospheres Data Node |  |  |
| PDS Atmospheres Node Manager Dr. Reta Beebe | Astronomy Department New Mexico State University P.O. Box 30001, MSC 4500 Las Cruces, NM 88003 | 575 646-1938 rbeebe@nmsu.edu |
| PDS Atmospheres Archive Manager Mr. Lyle Huber | Astronomy Department New Mexico State University P.O. Box 30001, MSC 4500 Las Cruces, NM 88003 | 575 646-1862 lhuber@nmsu.edu |

# REFERENCES

[1] Mahaffy, P.; Veverka, J.; Niemann, H.; Harpold, D.; Chiu, M.; Reynolds, E.; Owen, T.; Kasprzak, W.; Raaen, E.; Patrick, E.; Demick, J. (2001), An Overview of the Comet Nucleus TOUR Discovery Mission and a Description of Neutral Gas and Ion Measurements Planned, American Astronomical Society, DPS Meeting #33, #57.21; Bulletin of the American Astronomical Society, Vol. 33, p.1148.

[2] Waite, J. H.; Lewis, W. S.; Kasprzak, W. T.; Anicich, V. G.; Block, B. P.; Cravens, T. E.; Fletcher, G. G.; Ip, W.-H.; Luhmann, J. G.; McNutt, R. L.; Niemann, H. B.; Parejko, J. K.; Richards, J. E.; Thorpe, R. L.; Walter, E. M.; Yelle, R. V. (2004), The Cassini Ion and Neutral Mass Spectrometer (INMS) Investigation, Space Science Reviews, Volume 114, Issue 1-4, pp. 113-231.

# APPENDICES

## NMS Electrodes Designation

Table A-1: NMS Electrode list and designation (see Figure 3)

| Lens # | Designation | Abbreviation | Min Potential (V) | Max Potential (V) |
| --- | --- | --- | --- | --- |
| Open Source (OS) Sub Assembly |  |  |  |  |
| 1 | OS Collimator (a) (Split) | OS_COLa | -150 | 150 |
| 2 | OS Collimator (b) (Split) | OS_COLb | -150 | 150 |
| 3 | OS Lens 1 | OS_OL1 | -10 | 10 |
| 4 | OS Lens 2 | OS_OL2 | -10 | 10 |
| 5 | OS Lens 3 | OS_OL3 | -150 | 150 |
| 6 | OS Lens 4 | OS_OL4 | -300 | 0 |
| 7 | OS Lens 5 | OS_OL5 | -900 | 100 |
| 8 | OS Nozzle | OS_OL6 | -900 | 100 |
| 9 | OS Electron Restrictor 1 | OS_RES1 | -150 | 150 |
| 10 | OS Electron Deflector 1 (c) | OS_EF1c | -100 | 0 |
| 11 | OS Electron Deflector 1 (d) | OS_EF1d | -100 | 0 |
| 12 | OS Electron Focus 1 (a) | OS_EF1a | -100 | 0 |
| 13 | OS Electron Focus 1 (b) | OS_EF1b | -100 | 0 |
| 14 | OS Filament Shield 1 | OS_FS1 | -100 | 0 |
| 15 | OS Filament 1-P | OS_FIL1-P | -70 | -70 |
| 16 | OS Filament 1-M | OS_FIL1-M | -70 | -70 |
| 17 | OS Electron Accelerator 1 | OS_EA1 | -100 | 200 |
| 18 | OS Electron Restrictor 2 | OS_RES2 | -150 | 150 |
| 19 | OS Electron Deflector 2 (c) | OS_EF2c | -100 | 0 |
| 20 | OS Electron Deflector 2 (d) | OS_EF2d | -100 | 0 |
| 21 | OS Electron Focus 2 (a) | OS_EF2a | -100 | 0 |
| 22 | OS Electron Focus 2 (b) | OS_EF2b | -100 | 0 |
| 23 | OS Filament Shield 2 | OS_FS2 | -100 | 0 |
| 24 | OS Filament 2-P | OS_FIL2-P | -70 | -70 |
| 25 | OS Filament 2-M | OS_FIL2-M | -70 | -70 |
| 26 | OS Electron Accelerator 2 | OS_EA2 | -100 | 200 |
| Closed Source (CS) Sub Assembly |  |  |  |  |
| 27 | CS Ion Accelerator | CS_IA | -10 | 10 |
| 28 | CS Ion Focus a | CS_IFa | -300 | 0 |
| 29 | CS Ion Focus b | CS_IFb | -300 | 0 |
| 30 | CS Nozzle | CS_NZ | -300 | 0 |
| 31 | CS Repeller | CS_RP | -10 | 10 |
| 32 | CS Repeller Shield | CS_RS | -10 | 10 |
| 33 | CS Anode 1 | CS_AN1 | 0 | 300 |
| 34 | CS Electron Accelerator 1 | CS_EA1 | 0 | 300 |
| 35 | CS Electron Focus 1 (a) | CS_EF1a | -100 | 0 |
| 36 | CS Electron Focus 1 (b) | CS_EF1b | -100 | 0 |
| 37 | CS Electron Deflector 1 (c) | CS_EF1c | -100 | 0 |
| 38 | CS Electron Deflector 1 (d) | CS_EF1d | -100 | 0 |
| 39 | CS Filament 1-P | CS_FIL1-P | -70 | -70 |
| 40 | CS Filament 1-M | CS_FIL1-M | -70 | -70 |
| 41 | CS Filament Shield 1 | CS_FS1 | -100 | 0 |
| 42 | CS Anode 2 | CS_AN2 | 0 | 300 |
| 43 | CS Electron Accelerator 2 | CS_EA2 | 0 | 300 |
| 44 | CS Electron Focus 2 (a) | CS_EF2a | -100 | 0 |
| 45 | CS Electron Focus 2 (b) | CS_EF2b | -100 | 0 |
| 46 | CS Electron Deflector 2 (c) | CS_EF2c | -100 | 0 |
| 47 | CS Electron Deflector 2 (d) | CS_EF2d | -100 | 0 |
| 48 | CS Filament 2-P | CS_FIL2-P | -70 | -70 |
| 49 | CS Filament 2-M | CS_FIL2-M | -70 | -70 |
| 50 | CS Filament Shield 2 | CS_FS2 | -100 | 0 |
| Switching Lens (SL) Sub Assembly |  |  |  |  |
| 51 | Sl Quad Lens Top Front | SL_TF | -200 | 100 |
| 52 | SL Quad Lens Top Back | SL_TB | -900 | 100 |
| 53 | SL Quad Lens Bottom Back | SL_BB | -200 | 100 |
| 54 | Sl Quad Lens Bottom Front | SL_BF | -900 | 100 |
| 55 | SL End Lens 1 | SL_EL1 | -100 | 900 |
| 56 | SL End Lens 2 | SL_EL2 | -100 | 900 |
| Ion Analyzer (IA) Sub Assembly |  |  |  |  |
| 57 | IA Lens 1 | IA_L1 | -900 | 0 |
| 58 | IA Lens 2 | IA_L2 | -900 | 0 |
| 59 | IA Lens 4 (a) (Split) | IA_L4a | -300 | 0 |
| 60 | IA Lens 4 (b) (Split) | IA_L4b | -300 | 0 |
| 61 | IA Lens 5 (a) (Split) | IA_L5a | -900 | 0 |
| 62 | IA Lens 5 (b) (Split) | IA_L5b | -900 | 0 |
| 63 | IA Lens 6 | IA_L6 | -900 | 0 |
| Quadrupole (QD) Sub Assembly |  |  |  |  |
| 64 | Rod 1 | QD_R1 | RF | RF |
| 65 | Rod 2 | QD_R2 | RF | RF |
| 66 | Rod 3 | QD_R3 | RF | RF |
| 67 | Rod 4 | QD_R4 | RF | RF |
| 68 | Beam Shaping Lens | QD_BS | -150 | 150 |
| Multiplier (MT) Sub Assembly |  |  |  |  |
| 69 | Einzel Lens | MT_EZ | -200 | 100 |
| 70 | Mask 1 | MT_MA1 | -200 | 100 |
| 71 | Mask 2 | MT_MA2 | -200 | 100 |
| 72 | Window | MT_WD | -200 | 100 |
| 73 | Multiplier Neg 1 | MT_MU1 | -3500 | 0 |
| 74 | Multiplier Neg 2 | MT_MU2 | -3500 | 0 |
| 75 | Faraday Cup | MT_FC | -150 | 150 |

## NMS DAC ID designation:

This table provides the ID number of all Digital to Analog Converters (DAC) that can be displayed in the science data tables under DAC_ID (Table A-4 and A-6).

Table A-2: NMS Electrode list and designation (see Figure 3)

| DAC Designation | DAC ID |
| --- | --- |
| 16 | OS_FIL1_VCTL |
| 17 | OS_FIL1_ECTL |
| 18 | OS_FS1_VCTL |
| 19 | OS_FIL2_VCTL |
| 20 | OS_FIL2_ECTL |
| 21 | OS_FS2_VCTL |
| 22 | CS_FIL1_VCTL |
| 23 | CS_FIL1_ECTL |
| 24 | CS_FS1_VCTL |
| 25 | CS_FIL2_VCTL |
| 26 | CS_FIL2_ECTL |
| 27 | CS_FS2_VCTL |
| 28 | DT1_VCTL |
| 29 | DT2_VCTL |
| 30 | QB_VCTL |
| 32 | FIL_ON_CTRL |
| 34 | ROD_AC |
| 36 | ROD_DC |
| 37 | RF_FREQ |
| 47 | FIL_SEL |
| 48 | BA_FIL_VCTL |
| 49 | CS_NZ_VCTL |
| 50 | MT_MU1_VCTL |
| 51 | MT_MU2_VCTL |
| 52 | CS_EA1_VCTL |
| 53 | CS_AN1_VCTL |
| 54 | CS_EA2_VCTL |
| 55 | CS_AN2_VCTL |
| 56 | QD_BS_VCTL |
| 57 | MT_EZ_VCTL |
| 58 | MT_MA1_VCTL |
| 59 | MT_MA2_VCTL |
| 60 | MT_WD_VCTL |
| 61 | MT_FC_VCTL |
| 62 | OS_EA1_VCTL |
| 63 | OS_RES1_VCTL |
| 64 | OS_EA2_VCTL |
| 65 | OS_RES2_VCTL |
| 66 | OS_OL1_VCTL |
| 67 | OS_OL2_VCTL |
| 68 | OS_COLA_VCTL |
| 69 | OS_COLB_VCTL |
| 70 | CS_IA_VCTL |
| 71 | CS_RP_VCTL |
| 72 | CS_RS_VCTL |
| 76 | IA_L1_VCTL |
| 77 | IA_L2_VCTL |
| 78 | IA_L4A_VCTL |
| 79 | IA_L4B_VCTL |
| 80 | IA_L5_VCTL |
| 81 | IA_L6_VCTL |
| 82 | OS_EF1A_VCTL |
| 83 | OS_EF1B_VCTL |
| 84 | OS_EF1C_VCTL |
| 85 | OS_EF1D_VCTL |
| 86 | OS_EF2A_VCTL |
| 87 | OS_EF2B_VCTL |
| 88 | OS_EF2C_VCTL |
| 89 | OS_EF2D_VCTL |
| 90 | CS_EF1A_VCTL |
| 91 | CS_EF1B_VCTL |
| 92 | CS_EF1C_VCTL |
| 93 | CS_EF1D_VCTL |
| 94 | CS_EF2A_VCTL |
| 95 | CS_EF2B_VCTL |
| 96 | CS_EF2C_VCTL |
| 97 | CS_EF2D_VCTL |
| 100 | OS_OL4_VCTL |
| 101 | CS_IFA_VCTL |
| 102 | CS_IFB_VCTL |
| 103 | SL_TB_VCTL |
| 104 | SL_BB_VCTL |
| 105 | SL_BF_VCTL |
| 106 | SL_EL_VCTL |
| 107 | OS_OL3_VCTL |
| 108 | SL_TF_VCTL |
| 109 | OS_OL5_VCTL |
| 110 | OS_OL6_VCTL |

## NMS Data Product Column Descriptions

# Raw housekeeping data table

This table contains the raw housekeeping packets values generated while the instrument is on.

Table A-3: Definition of the raw housekeeping data table

| # | Name | Format | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | TIME. | Real | s | N/A | SCLK timestamp of any corresponding observed value. |
| 2 | MKID | Integer | N/A | N/A | Marker ID of the current data point. Markers are tag numbers given to related set of measurements. |
| 3 | CDH:+13A_VMON | Real | V | 0 – 5 | Engineering value of +13A_VMON at TIME. |
| 4 | CDH:+13V_MON | Real | V | 0 – 5 | Engineering value of +13V_MON at TIME. |
| 5 | CDH:+15RF_VMON | Real | V | 0 – 5 | Engineering value of +15RF_VMON at TIME. |
| 6 | CDH:+160_VMON | Real | V | 0 – 5 | Engineering value of +160_VMON at TIME. |
| 7 | CDH:+5D_VMON | Real | V | 0 – 5 | Engineering value of +5D_VMON at TIME. |
| 8 | CDH:+5VREF_DAC | Real | V | 0 – 5 | Engineering value of +5VREF_DAC at TIME. |
| 9 | CDH:+80RF_VMON | Real | V | 0 – 5 | Engineering value of +80RF_VMON at TIME. |
| 10 | CDH:-13A_VMON | Real | V | 0 – 5 | Engineering value of -13A_VMON at TIME. |
| 11 | CDH:-13V_MON | Real | V | 0 – 5 | Engineering value of -13V_MON at TIME. |
| 12 | CDH:-15RF_VMON | Real | V | 0 – 5 | Engineering value of -15RF_VMON at TIME. |
| 13 | CDH:-160_VMON | Real | V | 0 – 5 | Engineering value of -160_VMON at TIME. |
| 14 | CDH:-5.7VREF | Real | V | 0 – 5 | Engineering value of -5.7VREF at TIME. |
| 15 | CDH:-5VREF_DAC | Real | V | 0 – 5 | Engineering value of -5VREF_DAC at TIME. |
| 16 | CDH:3.3V_IMON | Real | V | 0 – 5 | Engineering value of 3.3V_IMON at TIME. |
| 17 | CDH:5V_IMON | Real | V | 0 – 5 | Engineering value of 5V_IMON at TIME. |
| 18 | CDH:AGC_TMP | Real | V | 0 – 5 | Engineering value of AGC_TMP at TIME. This value captures the temperature of the RF AGC board. |
| 19 | CDH:ARM1_MON | Real | V | 0 – 5 | Engineering value of ARM1_MON at TIME. |
| 20 | CDH:ARM2_MON | Real | V | 0 – 5 | Engineering value of ARM2_MON at TIME. |
| 21 | CDH:BA_FIL_EMIS | Real | V | 0 – 5 | Engineering value of BA_FIL_EMIS at TIME. |
| 22 | CDH:BA_FIL_IMON | Real | V | 0 – 5 | Engineering value of BA_FIL_IMON at TIME. |
| 23 | CDH:BA_FIL_VMON | Real | V | 0 – 5 | Engineering value of BA_FIL_VMON at TIME. |
| 24 | CDH:BA_GRID_IMON | Real | V | 0 – 5 | Engineering value of BA_GRID_IMON at TIME. |
| 25 | CDH:BA_PRES | Real | V | 0 – 5 | Engineering value of BA_PRES at TIME. |
| 26 | CDH:CDH_+5VREF | Real | V | 0 – 5 | Engineering value of CDH_+5VREF at TIME. |
| 27 | CDH:CDH_-5VREF | Real | V | 0 – 5 | Engineering value of CDH_-5VREF at TIME. |
| 28 | CDH:CDH_2.5VMON | Real | V | 0 – 5 | Engineering value of CDH_2.5VMON at TIME. |
| 29 | CDH:CDH_3.3VMON | Real | V | 0 – 5 | Engineering value of CDH_3.3VMON at TIME. |
| 30 | CDH:CDH_TMP | Real | V | 0 – 5 | Engineering value of CDH_TMP at TIME. This value captures the temperature of the CDH board. |
| 31 | CDH:CS_+13A_MON | Real | V | 0 – 5 | Engineering value of CS_+13A_MON at TIME. |
| 32 | CDH:CS_+5REF_MON | Real | V | 0 – 5 | Engineering value of CS_+5REF_MON at TIME. |
| 33 | CDH:CS_-5REF_MON | Real | V | 0 – 5 | Engineering value of CS_-5REF_MON at TIME. |
| 34 | CDH:CS_AN1_MON | Real | V | 0 – 5 | Engineering value of CS_AN1_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_AN1 electrode. |
| 35 | CDH:CS_AN2_MON | Real | V | 0 – 5 | Engineering value of CS_AN2_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_AN2 electrode. |
| 36 | CDH:CS_EA1_MON | Real | V | 0 – 5 | Engineering value of CS_EA1_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_EA1 electrode. |
| 37 | CDH:CS_EA2_MON | Real | V | 0 – 5 | Engineering value of CS_EA2_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_EA2 electrode. |
| 38 | CDH:CS_EF1A_MON | Real | V | 0 – 5 | Engineering value of CS_EF1A_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_EF1A electrode. |
| 39 | CDH:CS_EF1B_MON | Real | V | 0 – 5 | Engineering value of CS_EF1B_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_EF1B electrode. |
| 40 | CDH:CS_EF1C_MON | Real | V | 0 – 5 | Engineering value of CS_EF1C_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_EF1C electrode. |
| 41 | CDH:CS_EF1D_MON | Real | V | 0 – 5 | Engineering value of CS_EF1D_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_EF1D electrode. |
| 42 | CDH:CS_EF2A_MON | Real | V | 0 – 5 | Engineering value of CS_EF2A_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_EF2A electrode. |
| 43 | CDH:CS_EF2B_MON | Real | V | 0 – 5 | Engineering value of CS_EF2B_MON at TIME. This value captures the drive circuit input to 0 – 5 control the voltage on the CS_ EF2B electrode. |
| 44 | CDH:CS_EF2C_MON | Real | V | 0 – 5 | Engineering value of CS_EF2C_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_EF2C electrode. |
| 45 | CDH:CS_EF2D_MON | Real | V | 0 – 5 | Engineering value of CS_EF2D_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_EF2D electrode. |
| 46 | CDH:CS_FIL1_IMON | Real | V | 0 – 5 | Engineering value of CS_FIL1_IMON at TIME. This value captures CS_FIL1 current. |
| 47 | CDH:CS_FIL1_VMON | Real | V | 0 – 5 | Engineering value of CS_FIL1_VMON at TIME. This value captures CS_FIL1 voltage. |
| 48 | CDH:CS_FIL2_IMON | Real | V | 0 – 5 | Engineering value of CS_FIL2_IMON at TIME. This value captures CS_FIL2 current. |
| 49 | CDH:CS_FIL2_VMON | Real | V | 0 – 5 | Engineering value of CS_FIL2_VMON at TIME. This value captures CS_FIL2 voltage. |
| 50 | CDH:CS_FIL_EMON | Real | V | 0 – 5 | Engineering value of CS_FIL_EMON at TIME. This value captures the emission value on the active CS filament. |
| 51 | CDH:CS_FIL_SEL_ST | Real | N/A | N/A | Engineering value of CS_FIL_SEL_ST at TIME. |
| 52 | CDH:CS_FS1_MON | Real | V | 0 – 5 | Engineering value of CS_FS1_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_FS1 electrode. |
| 53 | CDH:CS_FS2_MON | Real | V | 0 – 5 | Engineering value of CS_FS2_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_FS2 electrode. |
| 54 | CDH:CS_GND_REF | Real | V | 0 – 5 | Engineering value of CS_GND_REF at TIME. |
| 55 | CDH:CS_IA_MON | Real | V | 0 – 5 | Engineering value of CS_IA_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_IA electrode. |
| 56 | CDH:CS_IFA_MON | Real | V | 0 – 5 | Engineering value of CS_IFA_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_IFA electrode. |
| 57 | CDH:CS_IFB_MON | Real | V | 0 – 5 | Engineering value of CS_IFB_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_IFB electrode. |
| 58 | CDH:CS_NZ_MON | Real | V | 0 – 5 | Engineering value of CS_NZ_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_NZ electrode. |
| 59 | CDH:CS_RP_MON | Real | V | 0 – 5 | Engineering value of CS_RP_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_RP electrode. |
| 60 | CDH:CS_RS_MON | Real | V | 0 – 5 | Engineering value of CS_RS_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_RS electrode. |
| 61 | CDH:CS_SPARE | Real | N/A | N/A | Unused channel. |
| 62 | CDH:CS_TMP | Real | V | 0 – 5 | Engineering value of CS_TMP at TIME. This value captures the temperature of the CS board. |
| 63 | CDH:CS_TRAP_MON | Real | V | 0 – 5 | Engineering value of CS_TRAP_MON at TIME. |
| 64 | CDH:CTL_+13VMON | Real | V | 0 – 5 | Engineering value of CTL_+13VMON at TIME. |
| 65 | CDH:CTL_+2.5VMON | Real | V | 0 – 5 | Engineering value of CTL_+2.5VMON at TIME. |
| 66 | CDH:CTL_+3.3VMON | Real | V | 0 – 5 | Engineering value of CTL_+3.3VMON at TIME. |
| 67 | CDH:CTL_+4VMON | Real | V | 0 – 5 | Engineering value of CTL_+4VMON at TIME. |
| 68 | CDH:CTL_+5VMON | Real | V | 0 – 5 | Engineering value of CTL_+5VMON at TIME. |
| 69 | CDH:CTL_+5VREF | Real | V | 0 – 5 | Engineering value of CTL_+5VREF at TIME. |
| 70 | CDH:CTL_+6VMON | Real | V | 0 – 5 | Engineering value of CTL_+6VMON at TIME. |
| 71 | CDH:CTL_-13VMON | Real | V | 0 – 5 | Engineering value of CTL_-13VMON at TIME. |
| 72 | CDH:CTL_-5VREF | Real | V | 0 – 5 | Engineering value of CTL_-5VREF at TIME. |
| 73 | CDH:CTL_SPARE | Real | N/A | N/A | Unused channel. |
| 74 | CDH:CTL_TMP | Real | V | 0 – 5 | Engineering value of CTL_TMP at TIME. This value captures the temperature of the CTL board. |
| 75 | CDH:DET_TMP | Real | V | 0 – 5 | Engineering value of DET_TMP at TIME. This value captures the temperature of the DET board. |
| 76 | CDH:EM1_IMON | Real | V | 0 – 5 | Engineering value of EM1_IMON at TIME. This value captures the current drawn by Multiplier 1. |
| 77 | CDH:EM2_IMON | Real | V | 0 – 5 | Engineering value of EM2_IMON at TIME. This value captures the current drawn by Multiplier 2. |
| 78 | CDH:EXT_THERM1 | Real | V | 0 – 5 | Engineering value of EXT_THERM1 at TIME. |
| 79 | CDH:EXT_THERM2 | Real | V | 0 – 5 | Engineering value of EXT_THERM2 at TIME. |
| 80 | CDH:EXT_THERM3 | Real | V | 0 – 5 | Engineering value of EXT_THERM3 at TIME. |
| 81 | CDH:FLASH_VMON | Real | V | 0 – 5 | Engineering value of FLASH_VMON at TIME. |
| 82 | CDH:IA_L1_MON | Real | V | 0 – 5 | Engineering value of IA_L1_MON at TIME. This value captures the drive circuit input needed to control the voltage on the IA_L1 electrode. |
| 83 | CDH:IA_L2_MON | Real | V | 0 – 5 | Engineering value of IA_L2_MON at TIME. This value captures the drive circuit input needed to control the voltage on the IA_L2 electrode. |
| 84 | CDH:IA_L4A_MON | Real | V | 0 – 5 | Engineering value of IA_L4A_MON at TIME. This value captures the drive circuit input needed to control the voltage on the IA_L4A electrode. |
| 85 | CDH:IA_L4B_MON | Real | V | 0 – 5 | Engineering value of IA_L4B_MON at TIME. This value captures the drive circuit input needed to control the voltage on the IA_L4B electrode. |
| 86 | CDH:IA_L5_MON | Real | V | 0 – 5 | Engineering value of IA_L5_MON at TIME. This value captures the drive circuit input needed to control the voltage on the IA_L5 electrode. |
| 87 | CDH:IA_L6_MON | Real | V | 0 – 5 | Engineering value of IA_L6_MON at TIME. This value captures the drive circuit input needed to control the voltage on the IA_L6 electrode. |
| 88 | CDH:IF_+5REF_MON | Real | V | 0 – 5 | Engineering value of IF_+5REF_MON at TIME. |
| 89 | CDH:IF_-5REF_MON | Real | V | 0 – 5 | Engineering value of IF_-5REF_MON at TIME. |
| 90 | CDH:IF_GND_REF | Real | V | 0 – 5 | Engineering value of IF_GND_REF at TIME. |
| 91 | CDH:IF_TMP | Real | V | 0 – 5 | Engineering value of IF_TMP at TIME. This value captures the temperature of the IF board. |
| 92 | CDH:MT_EZ_MON | Real | V | 0 – 5 | Engineering value of MT_EZ_MON at TIME. This value captures the drive circuit input needed to control the voltage on the MT_EZ electrode. |
| 93 | CDH:MT_FC_MON | Real | V | 0 – 5 | Engineering value of MT_FC_MON at TIME. This value captures the drive circuit input needed to control the voltage on the MT_FC electrode. |
| 94 | CDH:MT_MA1_MON | Real | V | 0 – 5 | Engineering value of MT_MA1_MON at TIME. This value captures the drive circuit input needed to control the voltage on the MT_MA1 electrode. |
| 95 | CDH:MT_MA2_MON | Real | V | 0 – 5 | Engineering value of MT_MA2_MON at TIME. This value captures the drive circuit input needed to control the voltage on the MT_MA2 electrode. |
| 96 | CDH:MT_WD_MON | Real | V | 0 – 5 | Engineering value of MT_WD_MON at TIME. This value captures the drive circuit input needed to control the voltage on the MT_WD electrode. |
| 97 | CDH:MULTANA1 | Real | V | 0 – 5 | Engineering value of MULTANA1 at TIME. |
| 98 | CDH:MULTANA2 | Real | V | 0 – 5 | Engineering value of MULTANA2 at TIME. |
| 99 | CDH:OS_+13A_MON | Real | V | 0 – 5 | Engineering value of OS_+13A_MON at TIME. |
| 100 | CDH:OS_+5REF_MON | Real | V | 0 – 5 | Engineering value of OS_+5REF_MON at TIME. |
| 101 | CDH:OS_-5REF_MON | Real | V | 0 – 5 | Engineering value of OS_-5REF_MON at TIME. |
| 102 | CDH:OS_COLA_MON | Real | V | 0 – 5 | Engineering value of OS_COLA_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_COLA electrode. |
| 103 | CDH:OS_COLB_MON | Real | V | 0 – 5 | Engineering value of OS_COLB_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_COLB electrode. |
| 104 | CDH:OS_EA1_MON | Real | V | 0 – 5 | Engineering value of OS_EA1_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EA1 electrode. |
| 105 | CDH:OS_EA2_MON | Real | V | 0 – 5 | Engineering value of OS_EA2_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EA2 electrode. |
| 106 | CDH:OS_EF1A_MON | Real | V | 0 – 5 | Engineering value of OS_EF1A_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF1A electrode. |
| 107 | CDH:OS_EF1B_MON | Real | V | 0 – 5 | Engineering value of OS_EF1B_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF1B electrode. |
| 108 | CDH:OS_EF1C_MON | Real | V | 0 – 5 | Engineering value of OS_EF1C_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF1C electrode. |
| 109 | CDH:OS_EF1D_MON | Real | V | 0 – 5 | Engineering value of OS_EF1D_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF1D electrode. |
| 110 | CDH:OS_EF2A_MON | Real | V | 0 – 5 | Engineering value of OS_EF2A_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF2A electrode. |
| 111 | CDH:OS_EF2B_MON | Real | V | 0 – 5 | Engineering value of OS_EF2B_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF2B electrode. |
| 112 | CDH:OS_EF2C_MON | Real | V | 0 – 5 | Engineering value of OS_EF2C_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF2C electrode. |
| 113 | CDH:OS_EF2D_MON | Real | V | 0 – 5 | Engineering value of OS_EF2D_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF2D electrode. |
| 114 | CDH:OS_FIL1_IMON | Real | V | 0 – 5 | Engineering value of OS_FIL1_IMON at TIME. This value captures OS_FIL1 current. |
| 115 | CDH:OS_FIL1_VMON | Real | V | 0 – 5 | Engineering value of OS_FIL1_VMON at TIME. This value captures OS_FIL1 voltage. |
| 116 | CDH:OS_FIL2_IMON | Real | V | 0 – 5 | Engineering value of OS_FIL2_IMON at TIME. This value captures OS_FIL2 current. |
| 117 | CDH:OS_FIL2_VMON | Real | V | 0 – 5 | Engineering value of OS_FIL2_VMON at TIME. This value captures OS_FIL2 voltage. |
| 118 | CDH:OS_FIL_EMON | Real | V | 0 – 5 | Engineering value of OS_FIL_EMON at TIME. This value captures the emission value on the active OS filament. |
| 119 | CDH:OS_FIL_SEL_ST | Real | N/A | N/A | Engineering value of OS_FIL_SEL_ST at TIME. |
| 120 | CDH:OS_FS1_MON | Real | V | 0 – 5 | Engineering value of OS_FS1_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_FS1 electrode. |
| 121 | CDH:OS_FS2_MON | Real | V | 0 – 5 | Engineering value of OS_FS2_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_FS2 electrode. |
| 122 | CDH:OS_GND_REF | Real | V | 0 – 5 | Engineering value of OS_GND_REF at TIME. |
| 123 | CDH:OS_OL1_MON | Real | V | 0 – 5 | Engineering value of OS_OL1_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_OL1 electrode. |
| 124 | CDH:OS_OL2_MON | Real | V | 0 – 5 | Engineering value of OS_OL2_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_OL2 electrode. |
| 125 | CDH:OS_OL3_MON | Real | V | 0 – 5 | Engineering value of OS_OL3_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_OL3 electrode. |
| 126 | CDH:OS_OL4_MON | Real | V | 0 – 5 | Engineering value of OS_OL4_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_OL4 electrode. |
| 127 | CDH:OS_OL5_MON | Real | V | 0 – 5 | Engineering value of OS_OL5_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_OL5 electrode. |
| 128 | CDH:OS_OL6_MON | Real | V | 0 – 5 | Engineering value of OS_OL6_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_OL6 electrode. |
| 129 | CDH:OS_RES1_MON | Real | V | 0 – 5 | Engineering value of OS_RES1_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_RES1 electrode. |
| 130 | CDH:OS_RES2_MON | Real | V | 0 – 5 | Engineering value of OS_RES2_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_RES2 electrode. |
| 131 | CDH:OS_SPARE | Real | N/A | N/A | Unused channel. |
| 132 | CDH:OS_TMP | Real | V | 0 – 5 | Engineering value of OS_TMP at TIME. This value captures the temperature of the OS board. |
| 133 | CDH:OS_TRAP_MON | Real | V | 0 – 5 | Engineering value of OS_TRAP_MON at TIME. |
| 134 | CDH:PS_IMON | Real | V | 0 – 5 | Engineering value of PS_IMON at TIME. |
| 135 | CDH:PS_IMON_2 | Real | V | 0 – 5 | Engineering value of PS_IMON_2 at TIME. |
| 136 | CDH:PS_TMP | Real | V | 0 – 5 | Engineering value of PS_TMP at TIME. This value captures the temperature of the PS board. |
| 137 | CDH:PYRO1_MON | Real | V | 0 – 5 | Engineering value of PYRO1_MON at TIME. |
| 138 | CDH:PYRO2_MON | Real | V | 0 – 5 | Engineering value of PYRO2_MON at TIME. |
| 139 | CDH:PulseCounter | Real | CTS | N/A | Engineering value of PulseCounter at TIME. This value captures the number of counts detected with active multiplier during the duration of the integration period. |
| 140 | CDH:QD_BS_MON | Real | V | 0 – 5 | Engineering value of QD_BS_MON at TIME. This value captures the drive circuit input needed to control the voltage on the QD_BS electrode. |
| 141 | CDH:RF_AGC_MON | Real | V | 0 – 5 | Engineering value of RF_AGC_MON at TIME. |
| 142 | CDH:RF_Cntr | Real | N/A | N/A | Engineering value of RF_Cntr at TIME. This value captures the current RF frequency. |
| 143 | CDH:RF_TMP | Real | V | 0 – 5 | Engineering value of RF_TMP at TIM. This value captures the temperature of the RF board. |
| 144 | CDH:SL_BB_MON | Real | V | 0 – 5 | Engineering value of SL_BB_MON at TIME. This value captures the drive circuit input needed to control the voltage on the SL_BB electrode. |
| 145 | CDH:SL_BF_MON | Real | V | 0 – 5 | Engineering value of SL_BF_MON at TIME. This value captures the drive circuit input needed to control the voltage on the SL_BF electrode. |
| 146 | CDH:SL_EL_MON | Real | V | 0 – 5 | Engineering value of SL_EL_MON at TIME. This value captures the drive circuit input needed to control the voltage on the SL_EL electrode. |
| 147 | CDH:SL_TB_MON | Real | V | 0 – 5 | Engineering value of SL_TB_MON at TIME. This value captures the drive circuit input needed to control the voltage on the SL_TB electrode. |
| 148 | CDH:SL_TF_MON | Real | V | 0 – 5 | Engineering value of SL_TF_MON at TIME. This value captures the drive circuit input needed to control the voltage on the SL_TF electrode. |
| 149 | CDH:SPARE_0 | Real | N/A | N/A | Unused channel. |
| 150 | CDH:SPARE_1 | Real | N/A | N/A | Unused channel. |
| 151 | CDH:SPARE_2 | Real | N/A | N/A | Unused channel. |
| 152 | CDH:SPARE_3 | Real | N/A | N/A | Unused channel. |
| 153 | CDH:SPARE_4 | Real | N/A | N/A | Unused channel. |
| 154 | CDH:SPARE_5 | Real | N/A | N/A | Unused channel. |
| 155 | CDH:SPARE_6 | Real | N/A | N/A | Unused channel. |
| 156 | CDH:THERM_COM | Real | N/A | N/A | Engineering value of THERM_COM at TIME. |
| 157 | FSW:ALARM_LEVEL | Real | N/A | N/A | Engineering value of ALARM_LEVEL at TIME. |
| 158 | FSW:ALARM_STAT | Real | N/A | N/A | Engineering value of ALARM_STAT at TIME. |
| 159 | FSW:BAD_CMD_ERR | Real | N/A | N/A | Engineering value of BAD_CMD_ERR at TIME. |
| 160 | FSW:BAD_CMD_OP | Real | N/A | N/A | Engineering value of BAD_CMD_OP at TIME. |
| 161 | FSW:CODE_CSUM | Real | N/A | N/A | Engineering value of CODE_CSUM at TIME. |
| 162 | FSW:DWELL_MON_ADDR | Real | N/A | N/A | Engineering value of DWELL_MON_ADDR at TIME. |
| 163 | FSW:DWELL_MON_VAL | Real | N/A | N/A | Engineering value of DWELL_MON_VAL at TIME. |
| 164 | FSW:FSW_VER | Real | N/A | N/A | Engineering value of FSW_VER at TIME. |
| 165 | FSW:INST_MODE | Real | N/A | N/A | Engineering value of INST_MODE at TIME. |
| 166 | FSW:LARGEST_FREE_BLOCK | Real | N/A | N/A | Engineering value of LARGEST_FREE_BLOCK at TIME. |
| 167 | FSW:LAST_CMD | Real | N/A | N/A | Engineering value of LAST_CMD at TIME. |
| 168 | FSW:LAST_FILE_ID | Real | N/A | N/A | Engineering value of LAST_FILE_ID at TIME. |
| 169 | FSW:LAST_MARKER | Real | N/A | N/A | Engineering value of LAST_MARKER at TIME. |
| 170 | FSW:LAST_RESET | Real | N/A | N/A | Engineering value of LAST_RESET at TIME. |
| 171 | FSW:LIB_CSUM | Real | N/A | N/A | Engineering value of LIB_CSUM at TIME. |
| 172 | FSW:LIB_VER | Real | N/A | N/A | Engineering value of LIB_VER at TIME. |
| 173 | FSW:LOAD_TYPE | Real | N/A | N/A | Engineering value of LOAD_TYPE at TIME. |
| 174 | FSW:MEM_ALLOC | Real | N/A | N/A | Engineering value of MEM_ALLOC at TIME. |
| 175 | FSW:MEM_FREE | Real | N/A | N/A | Engineering value of MEM_FREE at TIME. |
| 176 | FSW:N_ALARMS | Real | N/A | N/A | Engineering value of N_ALARMS at TIME. |
| 177 | FSW:N_ALARM_ACTIVE | Real | N/A | N/A | Engineering value of N_ALARM_ACTIVE at TIME. |
| 178 | FSW:N_ALARM_ENAB | Real | N/A | N/A | Engineering value of N_ALARM_ENAB at TIME. |
| 179 | FSW:N_ALLOCS | Real | N/A | N/A | Engineering value of N_ALLOCS at TIME. |
| 180 | FSW:N_CMDS | Real | N/A | N/A | Engineering value of N_CMDS at TIME. |
| 181 | FSW:N_CMD_ERRS | Real | N/A | N/A | Engineering value of N_CMD_ERRS at TIME. |
| 182 | FSW:N_FREES | Real | N/A | N/A | Engineering value of N_FREES at TIME. |
| 183 | FSW:PKT_REV | Real | N/A | N/A | Engineering value of PKT_REV at TIME. |
| 184 | FSW:POWER_STAT | Real | N/A | N/A | Engineering value of POWER_STAT at TIME. |
| 185 | FSW:SCRIPT_CSUM | Real | N/A | N/A | Engineering value of SCRIPT_CSUM at TIME. |
| 186 | FSW:SCRIPT_ID | Real | N/A | N/A | Engineering value of SCRIPT_ID at TIME. |
| 187 | FSW:SCRIPT_MODE | Real | N/A | N/A | Engineering value of SCRIPT_MODE at TIME. |
| 188 | FSW:SCRIPT_VER | Real | N/A | N/A | Engineering value of SCRIPT_VER at TIME. |
| 189 | FSW:SIDE_A | Real | N/A | N/A | Engineering value of SIDE_A at TIME. |
| 190 | FSW:TELEM_MODE | Real | N/A | N/A | Engineering value of TELEM_MODE at TIME. |
| 191 | FSW:ZONE_ALERT | Real | N/A | N/A | Engineering value of ZONE_ALERT at TIME. |
| 192 | QMS:ADC_STATUS | Integer | N/A | N/A | Engineering value of ADC_STATUS at TIME. |
| 193 | QMS:AMUX1 | Integer | N/A | N/A | Engineering value of AMUX1 at TIME. |
| 194 | QMS:AMUX2 | Integer | N/A | N/A | Engineering value of AMUX2 at TIME. |
| 195 | QMS:AMUX_ADDR1 | Integer | N/A | N/A | Engineering value of AMUX_ADDR1 at TIME. |
| 196 | QMS:AMUX_ADDR2 | Integer | N/A | N/A | Engineering value of AMUX_ADDR2 at TIME. |
| 197 | QMS:AMUX_ADDR3 | Integer | N/A | N/A | Engineering value of AMUX_ADDR3 at TIME. |
| 198 | QMS:AMUX_ADDR4 | Integer | N/A | N/A | Engineering value of AMUX_ADDR4 at TIME. |
| 199 | QMS:BA_FIL_VCTL | Real | N/A | N/A | Engineering value of BA_FIL_VCTL at TIME. |
| 200 | QMS:COUNT1 | Integer | N/A | NA | Engineering value of COUNT1 at TIME. This value captures the number of counts detected with multiplier 1 during the duration of the integration period. |
| 201 | QMS:COUNT2 | Integer | N/A | NA | Engineering value of COUNT2 at TIME. This value captures the number of counts detected with multiplier 2 during the duration of the integration period. |
| 202 | QMS:CS_AN1_VCTL | Real | N/A | 0 – 256 | Engineering value of CS_AN1_VCTL at TIME. This value captures the DAC setting for the CS_AN1 electrode. |
| 203 | QMS:CS_AN2_VCTL | Real | N/A | 0 – 256 | Engineering value of CS_AN2_VCTL at TIME. This value captures the DAC setting for the CS_AN2 electrode. |
| 204 | QMS:CS_EA1_VCTL | Real | N/A | 0 – 256 | Engineering value of CS_EA1_VCTL at TIME. This value captures the DAC setting for the CS_EA1 electrode. |
| 205 | QMS:CS_EA2_VCTL | Real | N/A | 0 – 256 | Engineering value of CS_EA2_VCTL at TIME. This value captures the DAC setting for the CS_EA2 electrode. |
| 206 | QMS:CS_EF1A_VCTL | Real | N/A | 0 – 4096 | Engineering value of CS_EF1A_VCTL at TIME. This value captures the DAC setting for the CS_EF1A electrode. |
| 207 | QMS:CS_EF1B_VCTL | Real | N/A | 0 – 4096 | Engineering value of CS_EF1B_VCTL at TIME. This value captures the DAC setting for the CS_EF1B electrode. |
| 208 | QMS:CS_EF1C_VCTL | Real | N/A | 0 – 4096 | Engineering value of CS_EF1C_VCTL at TIME. This value captures the DAC setting for the CS_EF1C electrode. |
| 209 | QMS:CS_EF1D_VCTL | Real | N/A | 0 – 4096 | Engineering value of CS_EF1D_VCTL at TIME. This value captures the DAC setting for the CS_EF1D electrode. |
| 210 | QMS:CS_EF2A_VCTL | Real | N/A | 0 – 4096 | Engineering value of CS_EF2A_VCTL at TIME. This value captures the DAC setting for the CS_EF2A electrode. |
| 211 | QMS:CS_EF2B_VCTL | Real | N/A | 0 – 4096 | Engineering value of CS_EF2B_VCTL at TIME. This value captures the DAC setting for the CS_EF2B electrode. |
| 212 | QMS:CS_EF2C_VCTL | Real | N/A | 0 – 4096 | Engineering value of CS_EF2C_VCTL at TIME. This value captures the DAC setting for the CS_EF2C electrode. |
| 213 | QMS:CS_EF2D_VCTL | Real | N/A | 0 – 4096 | Engineering value of CS_EF2D_VCTL at TIME. This value captures the DAC setting for the CS_EF2D electrode. |
| 214 | QMS:CS_FIL1_ECTL | Real | N/A | 0 – 256 | Engineering value of CS_FIL1_ECTL at TIME. This value captures the DAC setting for the CS_FIL1 electrode. |
| 215 | QMS:CS_FIL1_VCTL | Real | N/A | 0 – 256 | Engineering value of CS_FIL1_VCTL at TIME. This value captures the DAC setting for the CS_FIL1 electrode. |
| 216 | QMS:CS_FIL2_ECTL | Real | N/A | 0 – 256 | Engineering value of CS_FIL2_ECTL at TIME. This value captures the DAC setting for the CS_FIL2 electrode. |
| 217 | QMS:CS_FIL2_VCTL | Real | N/A | 0 – 256 | Engineering value of CS_FIL2_VCTL at TIME. This value captures the DAC setting for the CS_FIL2 electrode. |
| 218 | QMS:CS_FS1_VCTL | Real | N/A | 0 – 256 | Engineering value of CS_FS1_VCTL at TIME. This value captures the DAC setting for the CS_FS1 electrode. |
| 219 | QMS:CS_FS2_VCTL | Real | N/A | 0 – 256 | Engineering value of CS_FS2_VCTL at TIME. This value captures the DAC setting for the CS_FS2 electrode. |
| 220 | QMS:CS_IA_VCTL | Real | N/A | 0 – 256 | Engineering value of CS_IA_VCTL at TIME. This value captures the DAC setting for the CS_IA electrode. |
| 221 | QMS:CS_IFA_VCTL | Real | N/A | 0 – 4096 | Engineering value of CS_IFA_VCTL at TIME. This value captures the DAC setting for the CS_IFA electrode. |
| 222 | QMS:CS_IFB_VCTL | Real | N/A | 0 – 4096 | Engineering value of CS_IFB_VCTL at TIME. This value captures the DAC setting for the CS_IFB electrode. |
| 223 | QMS:CS_NZ_VCTL | Real | N/A | 0 – 256 | Engineering value of CS_NZ_VCTL at TIME. This value captures the DAC setting for the CS_NZ electrode. |
| 224 | QMS:CS_RP_VCTL | Real | N/A | 0 – 256 | Engineering value of CS_RP_VCTL at TIME. This value captures the DAC setting for the CS_RP electrode. |
| 225 | QMS:CS_RS_VCTL | Real | N/A | 0 – 256 | Engineering value of CS_RS_VCTL at TIME. This value captures the DAC setting for the CS_RS electrode. |
| 226 | QMS:CTL_DIGITAL_STATUS | Integer | N/A | N/A | Engineering value of CTL_DIGITAL_STATUS at TIME. |
| 227 | QMS:CTL_ERROR_COUNT | Integer | N/A | N/A | Engineering value of CTL_ERROR_COUNT at TIME. |
| 228 | QMS:CTL_PACKET_COUNT | Integer | N/A | N/A | Engineering value of CTL_PACKET_COUNT at TIME. |
| 229 | QMS:CTL_SYNC_CODE | Integer | N/A | N/A | Engineering value of CTL_SYNC_CODE at TIME. |
| 230 | QMS:DAC12BSPARE1 | Real | N/A | N/A | Unused channel. |
| 231 | QMS:DAC12BSPARE2 | Real | N/A | N/A | Unused channel. |
| 232 | QMS:DAC12BSPARE3 | Real | N/A | N/A | Unused channel. |
| 233 | QMS:DAC12USPARE1 | Real | N/A | N/A | Unused channel. |
| 234 | QMS:DAC12USPARE2 | Real | N/A | N/A | Unused channel. |
| 235 | QMS:DAC16BSPARE1 | Real | N/A | N/A | Unused channel. |
| 236 | QMS:DAC16BSPARE2 | Real | N/A | N/A | Unused channel. |
| 237 | QMS:DAC8B_SPARE1 | Real | N/A | N/A | Unused channel. |
| 238 | QMS:DAC8B_SPARE2 | Real | N/A | N/A | Unused channel. |
| 239 | QMS:DAC8B_SPARE3 | Real | N/A | N/A | Unused channel. |
| 240 | QMS:DAC_SPARE | Real | N/A | N/A | Unused channel. |
| 241 | QMS:DT1_VCTL | Real | N/A | 0 – 256 | Engineering value of DT1_VCTL at TIME. This value captures the DAC setting for the DET1 discriminator. |
| 242 | QMS:DT2_VCTL | Real | N/A | 0 – 256 | Engineering value of DT2_VCTL at TIME. This value captures the DAC setting for the DET1 discriminator. |
| 243 | QMS:FIL_ON_CTRL | Real | N/A | N/A | Engineering value of FIL_ON_CTRL at TIME. |
| 244 | QMS:FIL_SELECT | Integer | N/A | N/A | Engineering value of FIL_SELECT at TIME. |
| 245 | QMS:IA_L1_VCTL | Real | N/A | 0 – 4096 | Engineering value of IA_L1_VCTL at TIME. This value captures the DAC setting for the IA_L1 electrode. |
| 246 | QMS:IA_L2_VCTL | Real | N/A | 0 – 4096 | Engineering value of IA_L2_VCTL at TIME. This value captures the DAC setting for the IA_L2 electrode. |
| 247 | QMS:IA_L4A_VCTL | Real | N/A | 0 – 4096 | Engineering value of IA_L4A_VCTL at TIME. This value captures the DAC setting for the IA_L4A electrode. |
| 248 | QMS:IA_L4B_VCTL | Real | N/A | 0 – 4096 | Engineering value of IA_L4B_VCTL at TIME. This value captures the DAC setting for the IA_L4B electrode. |
| 249 | QMS:IA_L5_VCTL | Real | N/A | 0 – 4096 | Engineering value of IA_L5_VCTL at TIME. This value captures the DAC setting for the IA_L5 electrode. |
| 250 | QMS:IA_L6_VCTL | Real | N/A | 0 – 4096 | Engineering value of IA_L6_VCTL at TIME. This value captures the DAC setting for the IA_L6 electrode. |
| 251 | QMS:IP_COUNT | Real | ms | N/A | Engineering value of IP_COUNT at TIME. This value captures the current integration period (IP) duration. |
| 252 | QMS:IP_SETUP | Real | ms | N/A | Engineering value of IP_SETUP at TIME. This value captures the current settling period duration. |
| 253 | QMS:MT_EZ_VCTL | Real | N/A | 0 – 256 | Engineering value of MT_EZ_VCTL at TIME. This value captures the DAC setting for the MT_EZ electrode. |
| 254 | QMS:MT_FC_VCTL | Real | N/A | 0 – 256 | Engineering value of MT_FC_VCTL at TIME. This value captures the DAC setting for the MT_FC electrode. |
| 255 | QMS:MT_MA1_VCTL | Real | N/A | 0 – 256 | Engineering value of MT_MA1_VCTL at TIME. This value captures the DAC setting for the MT_MA1 electrode. |
| 256 | QMS:MT_MA2_VCTL | Real | N/A | 0 – 256 | Engineering value of MT_MA2_VCTL at TIME. This value captures the DAC setting for the MT_MA2 electrode. |
| 257 | QMS:MT_MU1_VCTL | Real | N/A | 0 – 256 | Engineering value of MT_MU1_VCTL at TIME. This value captures the DAC setting for the MT_MU1 electrode. |
| 258 | QMS:MT_MU2_VCTL | Real | N/A | 0 – 256 | Engineering value of MT_MU2_VCTL at TIME. This value captures the DAC setting for the MT_MU2 electrode. |
| 259 | QMS:MT_WD_VCTL | Real | N/A | 0 – 256 | Engineering value of MT_WD_VCTL at TIME. This value captures the DAC setting for the MT_WD electrode. |
| 260 | QMS:OS_COLA_VCTL | Real | N/A | 0 – 256 | Engineering value of OS_COLA_VCTL at TIME. This value captures the DAC setting for the OS_COLA electrode. |
| 261 | QMS:OS_COLB_VCTL | Real | N/A | 0 – 256 | Engineering value of OS_COLB_VCTL at TIME. This value captures the DAC setting for the OS_COLB electrode. |
| 262 | QMS:OS_EA1_VCTL | Real | N/A | 0 – 256 | Engineering value of OS_EA1_VCTL at TIME. This value captures the DAC setting for the OS_EA1 electrode. |
| 263 | QMS:OS_EA2_VCTL | Real | N/A | 0 – 256 | Engineering value of OS_EA2_VCTL at TIME. This value captures the DAC setting for the OS_EA2 electrode. |
| 264 | QMS:OS_EF1A_VCTL | Real | N/A | 0 – 4096 | Engineering value of OS_EF1A_VCTL at TIME. This value captures the DAC setting for the OS_EF1A electrode. |
| 265 | QMS:OS_EF1B_VCTL | Real | N/A | 0 – 4096 | Engineering value of OS_EF1B_VCTL at TIME. This value captures the DAC setting for the OS_EF1B electrode. |
| 266 | QMS:OS_EF1C_VCTL | Real | N/A | 0 – 4096 | Engineering value of OS_EF1C_VCTL at TIME. This value captures the DAC setting for the OS_EF1C electrode. |
| 267 | QMS:OS_EF1D_VCTL | Real | N/A | 0 – 4096 | Engineering value of OS_EF1D_VCTL at TIME. This value captures the DAC setting for the OS_EF1D electrode. |
| 268 | QMS:OS_EF2A_VCTL | Real | N/A | 0 – 4096 | Engineering value of OS_EF2A_VCTL at TIME. This value captures the DAC setting for the OS_EF2A electrode. |
| 269 | QMS:OS_EF2B_VCTL | Real | N/A | 0 – 4096 | Engineering value of OS_EF2B_VCTL at TIME. This value captures the DAC setting for the OS_EF2B electrode. |
| 270 | QMS:OS_EF2C_VCTL | Real | N/A | 0 – 4096 | Engineering value of OS_EF2C_VCTL at TIME. This value captures the DAC setting for the OS_EF2C electrode. |
| 271 | QMS:OS_EF2D_VCTL | Real | N/A | 0 – 4096 | Engineering value of OS_EF2D_VCTL at TIME. This value captures the DAC setting for the OS_EF2D electrode. |
| 272 | QMS:OS_FIL1_ECTL | Real | N/A | 0 – 256 | Engineering value of OS_FIL1_ECTL at TIME. This value captures the DAC setting for the OS_FIL1 emission control. |
| 273 | QMS:OS_FIL1_VCTL | Real | N/A | 0 – 256 | Engineering value of OS_FIL1_VCTL at TIME. This value captures the DAC setting for the OS_FIL1 electrode. |
| 274 | QMS:OS_FIL2_ECTL | Real | N/A | 0 – 256 | Engineering value of OS_FIL2_ECTL at TIME. This value captures the DAC setting for the OS_FIL2 emission control. |
| 275 | QMS:OS_FIL2_VCTL | Real | N/A | 0 – 256 | Engineering value of OS_FIL2_VCTL at TIME. This value captures the DAC setting for the OS_FIL2 electrode. |
| 276 | QMS:OS_FS1_VCTL | Real | N/A | 0 – 256 | Engineering value of OS_FS1_VCTL at TIME. This value captures the DAC setting for the OS_FS1 electrode. |
| 277 | QMS:OS_FS2_VCTL | Real | N/A | 0 – 256 | Engineering value of OS_FS2_VCTL at TIME. This value captures the DAC setting for the OS_FS2 electrode. |
| 278 | QMS:OS_OL1_VCTL | Real | N/A | 0 – 256 | Engineering value of OS_OL1_VCTL at TIME. This value captures the DAC setting for the OS_OL1 electrode. |
| 279 | QMS:OS_OL2_VCTL | Real | N/A | 0 – 256 | Engineering value of OS_OL2_VCTL at TIME. This value captures the DAC setting for the OS_OL2 electrode. |
| 280 | QMS:OS_OL3_VCTL | Real | N/A | 0 – 4096 | Engineering value of OS_OL3_VCTL at TIME. This value captures the DAC setting for the OS_OL3 electrode. |
| 281 | QMS:OS_OL4_VCTL | Real | N/A | 0 – 4096 | Engineering value of OS_OL4_VCTL at TIME. This value captures the DAC setting for the OS_OL4 electrode. |
| 282 | QMS:OS_OL5_VCTL | Real | N/A | 0 – 4096 | Engineering value of OS_OL5_VCTL at TIME. This value captures the DAC setting for the OS_OL5 electrode. |
| 283 | QMS:OS_OL6_VCTL | Real | N/A | 0 – 4096 | Engineering value of OS_OL6_VCTL at TIME. This value captures the DAC setting for the OS_OL6 electrode. |
| 284 | QMS:OS_RES1_VCTL | Real | N/A | 0 – 256 | Engineering value of OS_RES1_VCTL at TIME. This value captures the DAC setting for the OS_RES1 electrode. |
| 285 | QMS:OS_RES2_VCTL | Real | N/A | 0 – 256 | Engineering value of OS_RES2_VCTL at TIME. This value captures the DAC setting for the OS_RES2 electrode. |
| 286 | QMS:QB_VCTL | Real | V | 0 – 256 | Engineering value of QB_VCTL at TIME. |
| 287 | QMS:QD_BS_VCTL | Real | N/A | 0 – 256 | Engineering value of QD_BS_VCTL at TIME. This value captures the DAC setting for the QD_BS electrode. |
| 288 | QMS:RF_FREQ | Real | N/A | N/A | Engineering value of RF_FREQ at TIME. |
| 289 | QMS:RF_FREQ_SET | Integer | N/A | N/A | Engineering value of RF_FREQ_SET at TIME. |
| 290 | QMS:RODAC_CTRL | Real | N/A | 0 – 65536 | Engineering value of RODAC_CTRL at TIME. This value captures the DAC setting for the RF AC amplitude. |
| 291 | QMS:RODDC_CTL | Real | N/A | 0 – 65536 | Engineering value of RODDC_CTL at TIME. This value captures the DAC setting for the RF DC amplitude. |
| 292 | QMS:SERIAL_NUM | Integer | N/A | N/A | Engineering value of SERIAL_NUM at TIME. |
| 293 | QMS:SERIAL_NUM_SET | Integer | N/A | N/A | Engineering value of SERIAL_NUM_SET at TIME. |
| 294 | QMS:SL_BB_VCTL | Real | N/A | 0 – 4096 | Engineering value of SL_BB_VCTL at TIME. This value captures the DAC setting for the SL_BB electrode. |
| 295 | QMS:SL_BF_VCTL | Real | N/A | 0 – 4096 | Engineering value of SL_BF_VCTL at TIME. This value captures the DAC setting for the SL_BF electrode. |
| 296 | QMS:SL_EL_VCTL | Real | N/A | 0 – 4096 | Engineering value of SL_EL_VCTL at TIME. This value captures the DAC setting for the SL_EL electrode. |
| 297 | QMS:SL_TB_VCTL | Real | N/A | 0 – 4096 | Engineering value of SL_TB_VCTL at TIME. This value captures the DAC setting for the SL_TB electrode. |
| 298 | QMS:SL_TF_VCTL | Real | N/A | 0 – 4096 | Engineering value of SL_TF_VCTL at TIME. This value captures the DAC setting for the SL_TF electrode. |
| 299 | QMS:SYNC_CODE | Integer | N/A | N/A | Engineering value of SYNC_CODE at TIME. |
| 300 | QMS:WAIT | Integer | N/A | N/A | Engineering value of WAIT at TIME. |
| 301 | TM:TMMarker | Real | N/A | N/A | Engineering value of TMMarker at TIME. This value captures the Marker ID of the current data point. Markers are tag numbers given to related set of measurements. |
| 302 | TM:TMMarkerText | Text | N/A | N/A | Engineering value of TMMarkerText at TIME. This value captures the current Marker description. |
| 303 | TM:TMSync | Real | N/A | N/A | Engineering value of TMSync at TIME. |
| 304 | TM:TMSystemID | Real | N/A | N/A | Engineering value of TMSystemID at TIME. |
| 305 | TM:TMTick | Real | N/A | N/A | Engineering value of TMTick at TIME. |

# Raw science data table

This table contains the raw science packets values generated while the instrument is in a science telemetry mode.

Table A-4: Definition of the raw science data table

| # | Name | Format | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | TIME. | Real | s | N/A | SCLK timestamp of any corresponding observed value. |
| 2 | MKID | Integer | N/A | N/A | Marker ID of the current data point. Markers are tag numbers given to related set of measurements. |
| 3 | IP | Real | s | N/A | Engineering value of IP at TIME. This value captures the current integration period (IP) duration. |
| 4 | TUNING | Integer | N/A | N/A | Engineering value of TUNING at TIME. This value captures the current focusing scheme of the sensor. |
| 5 | MASS | Real | M/Z | 0 – 150 | Engineering value of MASS at TIME. This value captures the current measured mass value. |
| 6 | COUNTS | Real | Hz | N/A | Engineering value of COUNTS at TIME. This value captures the number of counts detected with the active multiplier during the duration of the integration period. |
| 7 | DAC_ID | Integer | N/A | N/A | Engineering value of DAC_ID at TIME. This value captures the ID of DAC used during electrode voltage scan (See Table A-2). |
| 8 | DAC_VOLTAGE | Real | N/A | N/A | Engineering value of DAC_VOLTAGE at TIME. This value captures the voltage setting of the DAC_ID electrode during its voltage scan. |

# Raw message log

The message log is an ASCII file that contains the messages generated by the C&DH as it executes the script. These messages are time tagged (in seconds) to allow the data user to correlate the data to the tasks executed by the instrument.

# Calibrated housekeeping table

This table contains the calibrated housekeeping packets values generated while the instrument is on.

Table A-5: Definition of the calibrated housekeeping table

| # | Name | Format | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | TIME. | Real | s | N/A | SCLK timestamp of any corresponding observed value. |
| 2 | MKID | Integer | N/A | N/A | Marker ID of the current data point. Markers are tag numbers given to related set of measurements. |
| 3 | CDH:+13A_VMON | Real | V | N/A | Scientific value of +13A_VMON at TIME. |
| 4 | CDH:+13V_MON | Real | V | N/A | Scientific value of +13V_MON at TIME. |
| 5 | CDH:+15RF_VMON | Real | V | N/A | Scientific value of +15RF_VMON at TIME. |
| 6 | CDH:+160_VMON | Real | V | N/A | Scientific value of +160_VMON at TIME. |
| 7 | CDH:+5D_VMON | Real | V | N/A | Scientific value of +5D_VMON at TIME. |
| 8 | CDH:+5VREF_DAC | Real | V | N/A | Scientific value of +5VREF_DAC at TIME. |
| 9 | CDH:+80RF_VMON | Real | V | N/A | Scientific value of +80RF_VMON at TIME. |
| 10 | CDH:-13A_VMON | Real | V | N/A | Scientific value of -13A_VMON at TIME. |
| 11 | CDH:-13V_MON | Real | V | N/A | Scientific value of -13V_MON at TIME. |
| 12 | CDH:-15RF_VMON | Real | V | N/A | Scientific value of -15RF_VMON at TIME. |
| 13 | CDH:-160_VMON | Real | V | N/A | Scientific value of -160_VMON at TIME. |
| 14 | CDH:-5.7VREF | Real | V | N/A | Scientific value of -5.7VREF at TIME. |
| 15 | CDH:-5VREF_DAC | Real | V | N/A | Scientific value of -5VREF_DAC at TIME. |
| 16 | CDH:3.3V_IMON | Real | A | N/A | Scientific value of 3.3V_IMON at TIME. |
| 17 | CDH:5V_IMON | Real | A | N/A | Scientific value of 5V_IMON at TIME. |
| 18 | CDH:AGC_TMP | Real | C | N/A | Scientific value of AGC_TMP at TIME. This value captures the temperature of the RF AGC board. |
| 19 | CDH:ARM1_MON | Real | V | N/A | Scientific value of ARM1_MON at TIME. |
| 20 | CDH:ARM2_MON | Real | V | N/A | Scientific value of ARM2_MON at TIME. |
| 21 | CDH:BA_FIL_EMIS | Real | V | N/A | Scientific value of BA_FIL_EMIS at TIME. |
| 22 | CDH:BA_FIL_IMON | Real | V | N/A | Scientific value of BA_FIL_IMON at TIME. |
| 23 | CDH:BA_FIL_VMON | Real | V | N/A | Scientific value of BA_FIL_VMON at TIME. |
| 24 | CDH:BA_GRID_IMON | Real | V | N/A | Scientific value of BA_GRID_IMON at TIME. |
| 25 | CDH:BA_PRES | Real | V | N/A | Scientific value of BA_PRES at TIME. |
| 26 | CDH:CDH_+5VREF | Real | V | N/A | Scientific value of CDH_+5VREF at TIME. |
| 27 | CDH:CDH_-5VREF | Real | V | N/A | Scientific value of CDH_-5VREF at TIME. |
| 28 | CDH:CDH_2.5VMON | Real | V | N/A | Scientific value of CDH_2.5VMON at TIME. |
| 29 | CDH:CDH_3.3VMON | Real | V | N/A | Scientific value of CDH_3.3VMON at TIME. |
| 30 | CDH:CDH_TMP | Real | C | N/A | Scientific value of CDH_TMP at TIME. This value captures the temperature of the CDH board. |
| 31 | CDH:CS_+13A_MON | Real | V | N/A | Scientific value of CS_+13A_MON at TIME. |
| 32 | CDH:CS_+5REF_MON | Real | V | N/A | Scientific value of CS_+5REF_MON at TIME. |
| 33 | CDH:CS_-5REF_MON | Real | V | N/A | Scientific value of CS_-5REF_MON at TIME. |
| 34 | CDH:CS_AN1_MON | Real | V | N/A | Scientific value of CS_AN1_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_AN1 electrode. |
| 35 | CDH:CS_AN2_MON | Real | V | N/A | Scientific value of CS_AN2_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_AN2 electrode. |
| 36 | CDH:CS_EA1_MON | Real | V | N/A | Scientific value of CS_EA1_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_EA1 electrode. |
| 37 | CDH:CS_EA2_MON | Real | V | N/A | Scientific value of CS_EA2_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_EA2 electrode. |
| 38 | CDH:CS_EF1A_MON | Real | V | N/A | Scientific value of CS_EF1A_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_EF1A electrode. |
| 39 | CDH:CS_EF1B_MON | Real | V | N/A | Scientific value of CS_EF1B_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_EF1B electrode. |
| 40 | CDH:CS_EF1C_MON | Real | V | N/A | Scientific value of CS_EF1C_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_EF1C electrode. |
| 41 | CDH:CS_EF1D_MON | Real | V | N/A | Scientific value of CS_EF1D_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_EF1D electrode. |
| 42 | CDH:CS_EF2A_MON | Real | V | N/A | Scientific value of CS_EF2A_MON at TIME. This value captures the drive circuit input to control the voltage on the CS_EF2A electrode. |
| 43 | CDH:CS_EF2B_MON | Real | V | N/A | Scientific value of CS_EF2B_MON at TIME. This value captures the drive circuit input to N/A control the voltage on the CS_ EF2B electrode. |
| 44 | CDH:CS_EF2C_MON | Real | V | N/A | Scientific value of CS_EF2C_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_EF2C electrode. |
| 45 | CDH:CS_EF2D_MON | Real | V | N/A | Scientific value of CS_EF2D_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_EF2D electrode. |
| 46 | CDH:CS_FIL1_IMON | Real | V | N/A | Scientific value of CS_FIL1_IMON at TIME. This value captures CS_FIL1 current. |
| 47 | CDH:CS_FIL1_VMON | Real | V | N/A | Scientific value of CS_FIL1_VMON at TIME. This value captures CS_FIL1 voltage. |
| 48 | CDH:CS_FIL2_IMON | Real | V | N/A | Scientific value of CS_FIL2_IMON at TIME. This value captures CS_FIL2 current. |
| 49 | CDH:CS_FIL2_VMON | Real | V | N/A | Scientific value of CS_FIL2_VMON at TIME. This value captures CS_FIL2 voltage. |
| 50 | CDH:CS_FIL_EMON | Real | V | N/A | Scientific value of CS_FIL_EMON at TIME. This value captures the emission value on the active CS filament. |
| 51 | CDH:CS_FIL_SEL_ST | Real | N/A | N/A | Scientific value of CS_FIL_SEL_ST at TIME. |
| 52 | CDH:CS_FS1_MON | Real | V | N/A | Scientific value of CS_FS1_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_FS1 electrode. |
| 53 | CDH:CS_FS2_MON | Real | V | N/A | Scientific value of CS_FS2_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_FS2 electrode. |
| 54 | CDH:CS_GND_REF | Real | V | N/A | Scientific value of CS_GND_REF at TIME. |
| 55 | CDH:CS_IA_MON | Real | V | N/A | Scientific value of CS_IA_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_IA electrode. |
| 56 | CDH:CS_IFA_MON | Real | V | N/A | Scientific value of CS_IFA_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_IFA electrode. |
| 57 | CDH:CS_IFB_MON | Real | V | N/A | Scientific value of CS_IFB_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_IFB electrode. |
| 58 | CDH:CS_NZ_MON | Real | V | N/A | Scientific value of CS_NZ_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_NZ electrode. |
| 59 | CDH:CS_RP_MON | Real | V | N/A | Scientific value of CS_RP_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_RP electrode. |
| 60 | CDH:CS_RS_MON | Real | V | N/A | Scientific value of CS_RS_MON at TIME. This value captures the drive circuit input needed to control the voltage on the CS_RS electrode. |
| 61 | CDH:CS_SPARE | Real | N/A | N/A | Unused channel. |
| 62 | CDH:CS_TMP | Real | C | N/A | Scientific value of CS_TMP at TIME. This value captures the temperature of the CS board. |
| 63 | CDH:CS_TRAP_MON | Real | V | N/A | Scientific value of CS_TRAP_MON at TIME. |
| 64 | CDH:CTL_+13VMON | Real | V | N/A | Scientific value of CTL_+13VMON at TIME. |
| 65 | CDH:CTL_+2.5VMON | Real | V | N/A | Scientific value of CTL_+2.5VMON at TIME. |
| 66 | CDH:CTL_+3.3VMON | Real | V | N/A | Scientific value of CTL_+3.3VMON at TIME. |
| 67 | CDH:CTL_+4VMON | Real | V | N/A | Scientific value of CTL_+4VMON at TIME. |
| 68 | CDH:CTL_+5VMON | Real | V | N/A | Scientific value of CTL_+5VMON at TIME. |
| 69 | CDH:CTL_+5VREF | Real | V | N/A | Scientific value of CTL_+5VREF at TIME. |
| 70 | CDH:CTL_+6VMON | Real | V | N/A | Scientific value of CTL_+6VMON at TIME. |
| 71 | CDH:CTL_-13VMON | Real | V | N/A | Scientific value of CTL_-13VMON at TIME. |
| 72 | CDH:CTL_-5VREF | Real | V | N/A | Scientific value of CTL_-5VREF at TIME. |
| 73 | CDH:CTL_SPARE | Real | N/A | N/A | Unused channel. |
| 74 | CDH:CTL_TMP | Real | C | N/A | Scientific value of CTL_TMP at TIME. This value captures the temperature of the CTL board. |
| 75 | CDH:DET_TMP | Real | C | N/A | Scientific value of DET_TMP at TIME. This value captures the temperature of the DET board. |
| 76 | CDH:EM1_IMON | Real | A | N/A | Scientific value of EM1_IMON at TIME. This value captures the current drawn by Multiplier 1. |
| 77 | CDH:EM2_IMON | Real | A | N/A | Scientific value of EM2_IMON at TIME. This value captures the current drawn by Multiplier 2. |
| 78 | CDH:EXT_THERM1 | Real | C | N/A | Scientific value of EXT_THERM1 at TIME. |
| 79 | CDH:EXT_THERM2 | Real | C | N/A | Scientific value of EXT_THERM2 at TIME. |
| 80 | CDH:EXT_THERM3 | Real | C | N/A | Scientific value of EXT_THERM3 at TIME. |
| 81 | CDH:FLASH_VMON | Real | V | N/A | Scientific value of FLASH_VMON at TIME. |
| 82 | CDH:IA_L1_MON | Real | V | N/A | Scientific value of IA_L1_MON at TIME. This value captures the drive circuit input needed to control the voltage on the IA_L1 electrode. |
| 83 | CDH:IA_L2_MON | Real | V | N/A | Scientific value of IA_L2_MON at TIME. This value captures the drive circuit input needed to control the voltage on the IA_L2 electrode. |
| 84 | CDH:IA_L4A_MON | Real | V | N/A | Scientific value of IA_L4A_MON at TIME. This value captures the drive circuit input needed to control the voltage on the IA_L4A electrode. |
| 85 | CDH:IA_L4B_MON | Real | V | N/A | Scientific value of IA_L4B_MON at TIME. This value captures the drive circuit input needed to control the voltage on the IA_L4B electrode. |
| 86 | CDH:IA_L5_MON | Real | V | N/A | Scientific value of IA_L5_MON at TIME. This value captures the drive circuit input needed to control the voltage on the IA_L5 electrode. |
| 87 | CDH:IA_L6_MON | Real | V | N/A | Scientific value of IA_L6_MON at TIME. This value captures the drive circuit input needed to control the voltage on the IA_L6 electrode. |
| 88 | CDH:IF_+5REF_MON | Real | V | N/A | Scientific value of IF_+5REF_MON at TIME. |
| 89 | CDH:IF_-5REF_MON | Real | V | N/A | Scientific value of IF_-5REF_MON at TIME. |
| 90 | CDH:IF_GND_REF | Real | V | N/A | Scientific value of IF_GND_REF at TIME. |
| 91 | CDH:IF_TMP | Real | C | N/A | Scientific value of IF_TMP at TIME. This value captures the temperature of the IF board. |
| 92 | CDH:MT_EZ_MON | Real | V | N/A | Scientific value of MT_EZ_MON at TIME. This value captures the drive circuit input needed to control the voltage on the MT_EZ electrode. |
| 93 | CDH:MT_FC_MON | Real | V | N/A | Scientific value of MT_FC_MON at TIME. This value captures the drive circuit input needed to control the voltage on the MT_FC electrode. |
| 94 | CDH:MT_MA1_MON | Real | V | N/A | Scientific value of MT_MA1_MON at TIME. This value captures the drive circuit input needed to control the voltage on the MT_MA1 electrode. |
| 95 | CDH:MT_MA2_MON | Real | V | N/A | Scientific value of MT_MA2_MON at TIME. This value captures the drive circuit input needed to control the voltage on the MT_MA2 electrode. |
| 96 | CDH:MT_WD_MON | Real | V | N/A | Scientific value of MT_WD_MON at TIME. This value captures the drive circuit input needed to control the voltage on the MT_WD electrode. |
| 97 | CDH:MULTANA1 | Real | V | N/A | Scientific value of MULTANA1 at TIME. |
| 98 | CDH:MULTANA2 | Real | V | N/A | Scientific value of MULTANA2 at TIME. |
| 99 | CDH:OS_+13A_MON | Real | V | N/A | Scientific value of OS_+13A_MON at TIME. |
| 100 | CDH:OS_+5REF_MON | Real | V | N/A | Scientific value of OS_+5REF_MON at TIME. |
| 101 | CDH:OS_-5REF_MON | Real | V | N/A | Scientific value of OS_-5REF_MON at TIME. |
| 102 | CDH:OS_COLA_MON | Real | V | N/A | Scientific value of OS_COLA_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_COLA electrode. |
| 103 | CDH:OS_COLB_MON | Real | V | N/A | Scientific value of OS_COLB_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_COLB electrode. |
| 104 | CDH:OS_EA1_MON | Real | V | N/A | Scientific value of OS_EA1_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EA1 electrode. |
| 105 | CDH:OS_EA2_MON | Real | V | N/A | Scientific value of OS_EA2_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EA2 electrode. |
| 106 | CDH:OS_EF1A_MON | Real | V | N/A | Scientific value of OS_EF1A_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF1A electrode. |
| 107 | CDH:OS_EF1B_MON | Real | V | N/A | Scientific value of OS_EF1B_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF1B electrode. |
| 108 | CDH:OS_EF1C_MON | Real | V | N/A | Scientific value of OS_EF1C_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF1C electrode. |
| 109 | CDH:OS_EF1D_MON | Real | V | N/A | Scientific value of OS_EF1D_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF1D electrode. |
| 110 | CDH:OS_EF2A_MON | Real | V | N/A | Scientific value of OS_EF2A_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF2A electrode. |
| 111 | CDH:OS_EF2B_MON | Real | V | N/A | Scientific value of OS_EF2B_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF2B electrode. |
| 112 | CDH:OS_EF2C_MON | Real | V | N/A | Scientific value of OS_EF2C_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF2C electrode. |
| 113 | CDH:OS_EF2D_MON | Real | V | N/A | Scientific value of OS_EF2D_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_EF2D electrode. |
| 114 | CDH:OS_FIL1_IMON | Real | A | N/A | Scientific value of OS_FIL1_IMON at TIME. This value captures OS_FIL1 current. |
| 115 | CDH:OS_FIL1_VMON | Real | V | N/A | Scientific value of OS_FIL1_VMON at TIME. This value captures OS_FIL1 voltage. |
| 116 | CDH:OS_FIL2_IMON | Real | A | N/A | Scientific value of OS_FIL2_IMON at TIME. This value captures OS_FIL2 current. |
| 117 | CDH:OS_FIL2_VMON | Real | V | N/A | Scientific value of OS_FIL2_VMON at TIME. This value captures OS_FIL2 voltage. |
| 118 | CDH:OS_FIL_EMON | Real | A | N/A | Scientific value of OS_FIL_EMON at TIME. This value captures the emission value on the active OS filament. |
| 119 | CDH:OS_FIL_SEL_ST | Real | N/A | N/A | Scientific value of OS_FIL_SEL_ST at TIME. |
| 120 | CDH:OS_FS1_MON | Real | V | N/A | Scientific value of OS_FS1_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_FS1 electrode. |
| 121 | CDH:OS_FS2_MON | Real | V | N/A | Scientific value of OS_FS2_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_FS2 electrode. |
| 122 | CDH:OS_GND_REF | Real | V | N/A | Scientific value of OS_GND_REF at TIME. |
| 123 | CDH:OS_OL1_MON | Real | V | N/A | Scientific value of OS_OL1_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_OL1 electrode. |
| 124 | CDH:OS_OL2_MON | Real | V | N/A | Scientific value of OS_OL2_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_OL2 electrode. |
| 125 | CDH:OS_OL3_MON | Real | V | N/A | Scientific value of OS_OL3_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_OL3 electrode. |
| 126 | CDH:OS_OL4_MON | Real | V | N/A | Scientific value of OS_OL4_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_OL4 electrode. |
| 127 | CDH:OS_OL5_MON | Real | V | N/A | Scientific value of OS_OL5_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_OL5 electrode. |
| 128 | CDH:OS_OL6_MON | Real | V | N/A | Scientific value of OS_OL6_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_OL6 electrode. |
| 129 | CDH:OS_RES1_MON | Real | V | N/A | Scientific value of OS_RES1_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_RES1 electrode. |
| 130 | CDH:OS_RES2_MON | Real | V | N/A | Scientific value of OS_RES2_MON at TIME. This value captures the drive circuit input needed to control the voltage on the OS_RES2 electrode. |
| 131 | CDH:OS_SPARE | Real | N/A | N/A | Unused channel. |
| 132 | CDH:OS_TMP | Real | C | N/A | Scientific value of OS_TMP at TIME. This value captures the temperature of the OS board. |
| 133 | CDH:OS_TRAP_MON | Real | V | N/A | Scientific value of OS_TRAP_MON at TIME. |
| 134 | CDH:PS_IMON | Real | A | N/A | Scientific value of PS_IMON at TIME. |
| 135 | CDH:PS_IMON_2 | Real | A | N/A | Scientific value of PS_IMON_2 at TIME. |
| 136 | CDH:PS_TMP | Real | C | N/A | Scientific value of PS_TMP at TIME. This value captures the temperature of the PS board. |
| 137 | CDH:PYRO1_MON | Real | V | N/A | Scientific value of PYRO1_MON at TIME. |
| 138 | CDH:PYRO2_MON | Real | V | N/A | Scientific value of PYRO2_MON at TIME. |
| 139 | CDH:PulseCounter | Real | Hz | N/A | Scientific value of PulseCounter at TIME. This value captures the number of counts detected with active multiplier during the duration of the integration period. |
| 140 | CDH:QD_BS_MON | Real | V | N/A | Scientific value of QD_BS_MON at TIME. This value captures the drive circuit input needed to control the voltage on the QD_BS electrode. |
| 141 | CDH:RF_AGC_MON | Real | V | N/A | Scientific value of RF_AGC_MON at TIME. |
| 142 | CDH:RF_Cntr | Real | KHz | N/A | Scientific value of RF_Cntr at TIME. This value captures the current RF frequency. |
| 143 | CDH:RF_TMP | Real | C | N/A | Scientific value of RF_TMP at TIM. This value captures the temperature of the RF board. |
| 144 | CDH:SL_BB_MON | Real | V | N/A | Scientific value of SL_BB_MON at TIME. This value captures the drive circuit input needed to control the voltage on the SL_BB electrode. |
| 145 | CDH:SL_BF_MON | Real | V | N/A | Scientific value of SL_BF_MON at TIME. This value captures the drive circuit input needed to control the voltage on the SL_BF electrode. |
| 146 | CDH:SL_EL_MON | Real | V | N/A | Scientific value of SL_EL_MON at TIME. This value captures the drive circuit input needed to control the voltage on the SL_EL electrode. |
| 147 | CDH:SL_TB_MON | Real | V | N/A | Scientific value of SL_TB_MON at TIME. This value captures the drive circuit input needed to control the voltage on the SL_TB electrode. |
| 148 | CDH:SL_TF_MON | Real | V | N/A | Scientific value of SL_TF_MON at TIME. This value captures the drive circuit input needed to control the voltage on the SL_TF electrode. |
| 149 | CDH:SPARE_0 | Real | N/A | N/A | Unused channel. |
| 150 | CDH:SPARE_1 | Real | N/A | N/A | Unused channel. |
| 151 | CDH:SPARE_2 | Real | N/A | N/A | Unused channel. |
| 152 | CDH:SPARE_3 | Real | N/A | N/A | Unused channel. |
| 153 | CDH:SPARE_4 | Real | N/A | N/A | Unused channel. |
| 154 | CDH:SPARE_5 | Real | N/A | N/A | Unused channel. |
| 155 | CDH:SPARE_6 | Real | N/A | N/A | Unused channel. |
| 156 | CDH:THERM_COM | Real | N/A | N/A | Scientific value of THERM_COM at TIME. |
| 157 | FSW:ALARM_LEVEL | Real | N/A | N/A | Scientific value of ALARM_LEVEL at TIME. |
| 158 | FSW:ALARM_STAT | Real | N/A | N/A | Scientific value of ALARM_STAT at TIME. |
| 159 | FSW:BAD_CMD_ERR | Real | N/A | N/A | Scientific value of BAD_CMD_ERR at TIME. |
| 160 | FSW:BAD_CMD_OP | Real | N/A | N/A | Scientific value of BAD_CMD_OP at TIME. |
| 161 | FSW:CODE_CSUM | Real | N/A | N/A | Scientific value of CODE_CSUM at TIME. |
| 162 | FSW:DWELL_MON_ADDR | Real | N/A | N/A | Scientific value of DWELL_MON_ADDR at TIME. |
| 163 | FSW:DWELL_MON_VAL | Real | N/A | N/A | Scientific value of DWELL_MON_VAL at TIME. |
| 164 | FSW:FSW_VER | Real | N/A | N/A | Scientific value of FSW_VER at TIME. |
| 165 | FSW:INST_MODE | Real | N/A | N/A | Scientific value of INST_MODE at TIME. |
| 166 | FSW:LARGEST_FREE_BLOCK | Real | N/A | N/A | Scientific value of LARGEST_FREE_BLOCK at TIME. |
| 167 | FSW:LAST_CMD | Real | N/A | N/A | Scientific value of LAST_CMD at TIME. |
| 168 | FSW:LAST_FILE_ID | Real | N/A | N/A | Scientific value of LAST_FILE_ID at TIME. |
| 169 | FSW:LAST_MARKER | Real | N/A | N/A | Scientific value of LAST_MARKER at TIME. |
| 170 | FSW:LAST_RESET | Real | N/A | N/A | Scientific value of LAST_RESET at TIME. |
| 171 | FSW:LIB_CSUM | Real | N/A | N/A | Scientific value of LIB_CSUM at TIME. |
| 172 | FSW:LIB_VER | Real | N/A | N/A | Scientific value of LIB_VER at TIME. |
| 173 | FSW:LOAD_TYPE | Real | N/A | N/A | Scientific value of LOAD_TYPE at TIME. |
| 174 | FSW:MEM_ALLOC | Real | N/A | N/A | Scientific value of MEM_ALLOC at TIME. |
| 175 | FSW:MEM_FREE | Real | N/A | N/A | Scientific value of MEM_FREE at TIME. |
| 176 | FSW:N_ALARMS | Real | N/A | N/A | Scientific value of N_ALARMS at TIME. |
| 177 | FSW:N_ALARM_ACTIVE | Real | N/A | N/A | Scientific value of N_ALARM_ACTIVE at TIME. |
| 178 | FSW:N_ALARM_ENAB | Real | N/A | N/A | Scientific value of N_ALARM_ENAB at TIME. |
| 179 | FSW:N_ALLOCS | Real | N/A | N/A | Scientific value of N_ALLOCS at TIME. |
| 180 | FSW:N_CMDS | Real | N/A | N/A | Scientific value of N_CMDS at TIME. |
| 181 | FSW:N_CMD_ERRS | Real | N/A | N/A | Scientific value of N_CMD_ERRS at TIME. |
| 182 | FSW:N_FREES | Real | N/A | N/A | Scientific value of N_FREES at TIME. |
| 183 | FSW:PKT_REV | Real | N/A | N/A | Scientific value of PKT_REV at TIME. |
| 184 | FSW:POWER_STAT | Real | N/A | N/A | Scientific value of POWER_STAT at TIME. |
| 185 | FSW:SCRIPT_CSUM | Real | N/A | N/A | Scientific value of SCRIPT_CSUM at TIME. |
| 186 | FSW:SCRIPT_ID | Real | N/A | N/A | Scientific value of SCRIPT_ID at TIME. |
| 187 | FSW:SCRIPT_MODE | Real | N/A | N/A | Scientific value of SCRIPT_MODE at TIME. |
| 188 | FSW:SCRIPT_VER | Real | N/A | N/A | Scientific value of SCRIPT_VER at TIME. |
| 189 | FSW:SIDE_A | Real | N/A | N/A | Scientific value of SIDE_A at TIME. |
| 190 | FSW:TELEM_MODE | Real | N/A | N/A | Scientific value of TELEM_MODE at TIME. |
| 191 | FSW:ZONE_ALERT | Real | N/A | N/A | Scientific value of ZONE_ALERT at TIME. |
| 192 | QMS:ADC_STATUS | Integer | N/A | N/A | Scientific value of ADC_STATUS at TIME. |
| 193 | QMS:AMUX1 | Integer | N/A | N/A | Scientific value of AMUX1 at TIME. |
| 194 | QMS:AMUX2 | Integer | N/A | N/A | Scientific value of AMUX2 at TIME. |
| 195 | QMS:AMUX_ADDR1 | Integer | N/A | N/A | Scientific value of AMUX_ADDR1 at TIME. |
| 196 | QMS:AMUX_ADDR2 | Integer | N/A | N/A | Scientific value of AMUX_ADDR2 at TIME. |
| 197 | QMS:AMUX_ADDR3 | Integer | N/A | N/A | Scientific value of AMUX_ADDR3 at TIME. |
| 198 | QMS:AMUX_ADDR4 | Integer | N/A | N/A | Scientific value of AMUX_ADDR4 at TIME. |
| 199 | QMS:BA_FIL_VCTL | Real | N/A | N/A | Scientific value of BA_FIL_VCTL at TIME. |
| 200 | QMS:COUNT1 | Integer | N/A | NA | Scientific value of COUNT1 at TIME. This value captures the number of counts detected with multiplier 1 during the duration of the integration period. |
| 201 | QMS:COUNT2 | Integer | N/A | NA | Scientific value of COUNT2 at TIME. This value captures the number of counts detected with multiplier 2 during the duration of the integration period. |
| 202 | QMS:CS_AN1_VCTL | Real | V | N/A | Scientific value of CS_AN1_VCTL at TIME. This value captures the DAC setting for the CS_AN1 electrode. |
| 203 | QMS:CS_AN2_VCTL | Real | V | N/A | Scientific value of CS_AN2_VCTL at TIME. This value captures the DAC setting for the CS_AN2 electrode. |
| 204 | QMS:CS_EA1_VCTL | Real | V | N/A | Scientific value of CS_EA1_VCTL at TIME. This value captures the DAC setting for the CS_EA1 electrode. |
| 205 | QMS:CS_EA2_VCTL | Real | V | N/A | Scientific value of CS_EA2_VCTL at TIME. This value captures the DAC setting for the CS_EA2 electrode. |
| 206 | QMS:CS_EF1A_VCTL | Real | V | N/A | Scientific value of CS_EF1A_VCTL at TIME. This value captures the DAC setting for the CS_EF1A electrode. |
| 207 | QMS:CS_EF1B_VCTL | Real | V | N/A | Scientific value of CS_EF1B_VCTL at TIME. This value captures the DAC setting for the CS_EF1B electrode. |
| 208 | QMS:CS_EF1C_VCTL | Real | V | N/A | Scientific value of CS_EF1C_VCTL at TIME. This value captures the DAC setting for the CS_EF1C electrode. |
| 209 | QMS:CS_EF1D_VCTL | Real | V | N/A | Scientific value of CS_EF1D_VCTL at TIME. This value captures the DAC setting for the CS_EF1D electrode. |
| 210 | QMS:CS_EF2A_VCTL | Real | V | N/A | Scientific value of CS_EF2A_VCTL at TIME. This value captures the DAC setting for the CS_EF2A electrode. |
| 211 | QMS:CS_EF2B_VCTL | Real | V | N/A | Scientific value of CS_EF2B_VCTL at TIME. This value captures the DAC setting for the CS_EF2B electrode. |
| 212 | QMS:CS_EF2C_VCTL | Real | V | N/A | Scientific value of CS_EF2C_VCTL at TIME. This value captures the DAC setting for the CS_EF2C electrode. |
| 213 | QMS:CS_EF2D_VCTL | Real | V | N/A | Scientific value of CS_EF2D_VCTL at TIME. This value captures the DAC setting for the CS_EF2D electrode. |
| 214 | QMS:CS_FIL1_ECTL | Real | A | N/A | Scientific value of CS_FIL1_ECTL at TIME. This value captures the DAC setting for the CS_FIL1 electrode. |
| 215 | QMS:CS_FIL1_VCTL | Real | V | N/A | Scientific value of CS_FIL1_VCTL at TIME. This value captures the DAC setting for the CS_FIL1 electrode. |
| 216 | QMS:CS_FIL2_ECTL | Real | A | N/A | Scientific value of CS_FIL2_ECTL at TIME. This value captures the DAC setting for the CS_FIL2 electrode. |
| 217 | QMS:CS_FIL2_VCTL | Real | V | N/A | Scientific value of CS_FIL2_VCTL at TIME. This value captures the DAC setting for the CS_FIL2 electrode. |
| 218 | QMS:CS_FS1_VCTL | Real | V | N/A | Scientific value of CS_FS1_VCTL at TIME. This value captures the DAC setting for the CS_FS1 electrode. |
| 219 | QMS:CS_FS2_VCTL | Real | V | N/A | Scientific value of CS_FS2_VCTL at TIME. This value captures the DAC setting for the CS_FS2 electrode. |
| 220 | QMS:CS_IA_VCTL | Real | V | N/A | Scientific value of CS_IA_VCTL at TIME. This value captures the DAC setting for the CS_IA electrode. |
| 221 | QMS:CS_IFA_VCTL | Real | V | N/A | Scientific value of CS_IFA_VCTL at TIME. This value captures the DAC setting for the CS_IFA electrode. |
| 222 | QMS:CS_IFB_VCTL | Real | V | N/A | Scientific value of CS_IFB_VCTL at TIME. This value captures the DAC setting for the CS_IFB electrode. |
| 223 | QMS:CS_NZ_VCTL | Real | V | N/A | Scientific value of CS_NZ_VCTL at TIME. This value captures the DAC setting for the CS_NZ electrode. |
| 224 | QMS:CS_RP_VCTL | Real | V | N/A | Scientific value of CS_RP_VCTL at TIME. This value captures the DAC setting for the CS_RP electrode. |
| 225 | QMS:CS_RS_VCTL | Real | V | N/A | Scientific value of CS_RS_VCTL at TIME. This value captures the DAC setting for the CS_RS electrode. |
| 226 | QMS:CTL_DIGITAL_STATUS | Integer | N/A | N/A | Scientific value of CTL_DIGITAL_STATUS at TIME. |
| 227 | QMS:CTL_ERROR_COUNT | Integer | N/A | N/A | Scientific value of CTL_ERROR_COUNT at TIME. |
| 228 | QMS:CTL_PACKET_COUNT | Integer | N/A | N/A | Scientific value of CTL_PACKET_COUNT at TIME. |
| 229 | QMS:CTL_SYNC_CODE | Integer | N/A | N/A | Scientific value of CTL_SYNC_CODE at TIME. |
| 230 | QMS:DAC12BSPARE1 | Real | N/A | N/A | Unused channel. |
| 231 | QMS:DAC12BSPARE2 | Real | N/A | N/A | Unused channel. |
| 232 | QMS:DAC12BSPARE3 | Real | N/A | N/A | Unused channel. |
| 233 | QMS:DAC12USPARE1 | Real | N/A | N/A | Unused channel. |
| 234 | QMS:DAC12USPARE2 | Real | N/A | N/A | Unused channel. |
| 235 | QMS:DAC16BSPARE1 | Real | N/A | N/A | Unused channel. |
| 236 | QMS:DAC16BSPARE2 | Real | N/A | N/A | Unused channel. |
| 237 | QMS:DAC8B_SPARE1 | Real | N/A | N/A | Unused channel. |
| 238 | QMS:DAC8B_SPARE2 | Real | N/A | N/A | Unused channel. |
| 239 | QMS:DAC8B_SPARE3 | Real | N/A | N/A | Unused channel. |
| 240 | QMS:DAC_SPARE | Real | N/A | N/A | Unused channel. |
| 241 | QMS:DT1_VCTL | Real | N/A | N/A | Scientific value of DT1_VCTL at TIME. This value captures the DAC setting for the DET1 discriminator. |
| 242 | QMS:DT2_VCTL | Real | N/A | N/A | Scientific value of DT2_VCTL at TIME. This value captures the DAC setting for the DET1 discriminator. |
| 243 | QMS:FIL_ON_CTRL | Real | N/A | N/A | Scientific value of FIL_ON_CTRL at TIME. |
| 244 | QMS:FIL_SELECT | Integer | N/A | N/A | Scientific value of FIL_SELECT at TIME. |
| 245 | QMS:IA_L1_VCTL | Real | V | N/A | Scientific value of IA_L1_VCTL at TIME. This value captures the DAC setting for the IA_L1 electrode. |
| 246 | QMS:IA_L2_VCTL | Real | V | N/A | Scientific value of IA_L2_VCTL at TIME. This value captures the DAC setting for the IA_L2 electrode. |
| 247 | QMS:IA_L4A_VCTL | Real | V | N/A | Scientific value of IA_L4A_VCTL at TIME. This value captures the DAC setting for the IA_L4A electrode. |
| 248 | QMS:IA_L4B_VCTL | Real | V | N/A | Scientific value of IA_L4B_VCTL at TIME. This value captures the DAC setting for the IA_L4B electrode. |
| 249 | QMS:IA_L5_VCTL | Real | V | N/A | Scientific value of IA_L5_VCTL at TIME. This value captures the DAC setting for the IA_L5 electrode. |
| 250 | QMS:IA_L6_VCTL | Real | V | N/A | Scientific value of IA_L6_VCTL at TIME. This value captures the DAC setting for the IA_L6 electrode. |
| 251 | QMS:IP_COUNT | Real | ms | N/A | Scientific value of IP_COUNT at TIME. This value captures the current integration period (IP) duration. |
| 252 | QMS:IP_SETUP | Real | ms | N/A | Scientific value of IP_SETUP at TIME. This value captures the current settling period duration. |
| 253 | QMS:MT_EZ_VCTL | Real | V | N/A | Scientific value of MT_EZ_VCTL at TIME. This value captures the DAC setting for the MT_EZ electrode. |
| 254 | QMS:MT_FC_VCTL | Real | V | N/A | Scientific value of MT_FC_VCTL at TIME. This value captures the DAC setting for the MT_FC electrode. |
| 255 | QMS:MT_MA1_VCTL | Real | V | N/A | Scientific value of MT_MA1_VCTL at TIME. This value captures the DAC setting for the MT_MA1 electrode. |
| 256 | QMS:MT_MA2_VCTL | Real | V | N/A | Scientific value of MT_MA2_VCTL at TIME. This value captures the DAC setting for the MT_MA2 electrode. |
| 257 | QMS:MT_MU1_VCTL | Real | V | N/A | Scientific value of MT_MU1_VCTL at TIME. This value captures the DAC setting for the MT_MU1 electrode. |
| 258 | QMS:MT_MU2_VCTL | Real | V | N/A | Scientific value of MT_MU2_VCTL at TIME. This value captures the DAC setting for the MT_MU2 electrode. |
| 259 | QMS:MT_WD_VCTL | Real | V | N/A | Scientific value of MT_WD_VCTL at TIME. This value captures the DAC setting for the MT_WD electrode. |
| 260 | QMS:OS_COLA_VCTL | Real | V | N/A | Scientific value of OS_COLA_VCTL at TIME. This value captures the DAC setting for the OS_COLA electrode. |
| 261 | QMS:OS_COLB_VCTL | Real | V | N/A | Scientific value of OS_COLB_VCTL at TIME. This value captures the DAC setting for the OS_COLB electrode. |
| 262 | QMS:OS_EA1_VCTL | Real | V | N/A | Scientific value of OS_EA1_VCTL at TIME. This value captures the DAC setting for the OS_EA1 electrode. |
| 263 | QMS:OS_EA2_VCTL | Real | V | N/A | Scientific value of OS_EA2_VCTL at TIME. This value captures the DAC setting for the OS_EA2 electrode. |
| 264 | QMS:OS_EF1A_VCTL | Real | V | N/A | Scientific value of OS_EF1A_VCTL at TIME. This value captures the DAC setting for the OS_EF1A electrode. |
| 265 | QMS:OS_EF1B_VCTL | Real | V | N/A | Scientific value of OS_EF1B_VCTL at TIME. This value captures the DAC setting for the OS_EF1B electrode. |
| 266 | QMS:OS_EF1C_VCTL | Real | V | N/A | Scientific value of OS_EF1C_VCTL at TIME. This value captures the DAC setting for the OS_EF1C electrode. |
| 267 | QMS:OS_EF1D_VCTL | Real | V | N/A | Scientific value of OS_EF1D_VCTL at TIME. This value captures the DAC setting for the OS_EF1D electrode. |
| 268 | QMS:OS_EF2A_VCTL | Real | V | N/A | Scientific value of OS_EF2A_VCTL at TIME. This value captures the DAC setting for the OS_EF2A electrode. |
| 269 | QMS:OS_EF2B_VCTL | Real | V | N/A | Scientific value of OS_EF2B_VCTL at TIME. This value captures the DAC setting for the OS_EF2B electrode. |
| 270 | QMS:OS_EF2C_VCTL | Real | V | N/A | Scientific value of OS_EF2C_VCTL at TIME. This value captures the DAC setting for the OS_EF2C electrode. |
| 271 | QMS:OS_EF2D_VCTL | Real | V | N/A | Scientific value of OS_EF2D_VCTL at TIME. This value captures the DAC setting for the OS_EF2D electrode. |
| 272 | QMS:OS_FIL1_ECTL | Real | A | N/A | Scientific value of OS_FIL1_ECTL at TIME. This value captures the DAC setting for the OS_FIL1 emission control. |
| 273 | QMS:OS_FIL1_VCTL | Real | V | N/A | Scientific value of OS_FIL1_VCTL at TIME. This value captures the DAC setting for the OS_FIL1 electrode. |
| 274 | QMS:OS_FIL2_ECTL | Real | A | N/A | Scientific value of OS_FIL2_ECTL at TIME. This value captures the DAC setting for the OS_FIL2 emission control. |
| 275 | QMS:OS_FIL2_VCTL | Real | V | N/A | Scientific value of OS_FIL2_VCTL at TIME. This value captures the DAC setting for the OS_FIL2 electrode. |
| 276 | QMS:OS_FS1_VCTL | Real | V | N/A | Scientific value of OS_FS1_VCTL at TIME. This value captures the DAC setting for the OS_FS1 electrode. |
| 277 | QMS:OS_FS2_VCTL | Real | V | N/A | Scientific value of OS_FS2_VCTL at TIME. This value captures the DAC setting for the OS_FS2 electrode. |
| 278 | QMS:OS_OL1_VCTL | Real | V | N/A | Scientific value of OS_OL1_VCTL at TIME. This value captures the DAC setting for the OS_OL1 electrode. |
| 279 | QMS:OS_OL2_VCTL | Real | V | N/A | Scientific value of OS_OL2_VCTL at TIME. This value captures the DAC setting for the OS_OL2 electrode. |
| 280 | QMS:OS_OL3_VCTL | Real | V | N/A | Scientific value of OS_OL3_VCTL at TIME. This value captures the DAC setting for the OS_OL3 electrode. |
| 281 | QMS:OS_OL4_VCTL | Real | V | N/A | Scientific value of OS_OL4_VCTL at TIME. This value captures the DAC setting for the OS_OL4 electrode. |
| 282 | QMS:OS_OL5_VCTL | Real | V | N/A | Scientific value of OS_OL5_VCTL at TIME. This value captures the DAC setting for the OS_OL5 electrode. |
| 283 | QMS:OS_OL6_VCTL | Real | V | N/A | Scientific value of OS_OL6_VCTL at TIME. This value captures the DAC setting for the OS_OL6 electrode. |
| 284 | QMS:OS_RES1_VCTL | Real | V | N/A | Scientific value of OS_RES1_VCTL at TIME. This value captures the DAC setting for the OS_RES1 electrode. |
| 285 | QMS:OS_RES2_VCTL | Real | V | N/A | Scientific value of OS_RES2_VCTL at TIME. This value captures the DAC setting for the OS_RES2 electrode. |
| 286 | QMS:QB_VCTL | Real | V | N/A | Scientific value of QB_VCTL at TIME. |
| 287 | QMS:QD_BS_VCTL | Real | V | N/A | Scientific value of QD_BS_VCTL at TIME. This value captures the DAC setting for the QD_BS electrode. |
| 288 | QMS:RF_FREQ | Real | N/A | N/A | Scientific value of RF_FREQ at TIME. |
| 289 | QMS:RF_FREQ_SET | Integer | N/A | N/A | Scientific value of RF_FREQ_SET at TIME. |
| 290 | QMS:RODAC_CTRL | Real | V | N/A | Scientific value of RODAC_CTRL at TIME. This value captures the DAC setting for the RF AC amplitude. |
| 291 | QMS:RODDC_CTL | Real | V | N/A | Scientific value of RODDC_CTL at TIME. This value captures the DAC setting for the RF DC amplitude. |
| 292 | QMS:SERIAL_NUM | Integer | N/A | N/A | Scientific value of SERIAL_NUM at TIME. |
| 293 | QMS:SERIAL_NUM_SET | Integer | N/A | N/A | Scientific value of SERIAL_NUM_SET at TIME. |
| 294 | QMS:SL_BB_VCTL | Real | V | N/A | Scientific value of SL_BB_VCTL at TIME. This value captures the DAC setting for the SL_BB electrode. |
| 295 | QMS:SL_BF_VCTL | Real | V | N/A | Scientific value of SL_BF_VCTL at TIME. This value captures the DAC setting for the SL_BF electrode. |
| 296 | QMS:SL_EL_VCTL | Real | V | N/A | Scientific value of SL_EL_VCTL at TIME. This value captures the DAC setting for the SL_EL electrode. |
| 297 | QMS:SL_TB_VCTL | Real | V | N/A | Scientific value of SL_TB_VCTL at TIME. This value captures the DAC setting for the SL_TB electrode. |
| 298 | QMS:SL_TF_VCTL | Real | V | N/A | Scientific value of SL_TF_VCTL at TIME. This value captures the DAC setting for the SL_TF electrode. |
| 299 | QMS:SYNC_CODE | Integer | N/A | N/A | Scientific value of SYNC_CODE at TIME. |
| 300 | QMS:WAIT | Integer | N/A | N/A | Scientific value of WAIT at TIME. |
| 301 | TM:TMMarker | Real | N/A | N/A | Scientific value of TMMarker at TIME. This value captures the Marker ID of the current data point. Markers are tag numbers given to related set of measurements. |
| 302 | TM:TMMarkerText | Text | N/A | N/A | Scientific value of TMMarkerText at TIME. This value captures the current Marker description. |
| 303 | TM:TMSync | Real | N/A | N/A | Scientific value of TMSync at TIME. |
| 304 | TM:TMSystemID | Real | N/A | N/A | Scientific value of TMSystemID at TIME. |
| 305 | TM:TMTick | Real | N/A | N/A | Scientific value of TMTick at TIME. |

# Calibrated science data table

This table contains the calibrated science packets values generated while the instrument is in a science telemetry mode.

Table A-6: Definition of the raw science data table

| # | Name | Format | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | TIME. | Real | s | N/A | SCLK timestamp of any corresponding observed value. |
| 2 | MKID | Integer | N/A | N/A | Marker ID of the current data point. Markers are tag numbers given to related set of measurements. |
| 3 | IP | Real | s | N/A | Scientific value of IP at TIME. This value captures the current integration period (IP) duration. |
| 4 | TUNING | Integer | N/A | N/A | Engineering value of TUNING at TIME. This value captures the current focusing scheme of the sensor. |
| 5 | MASS | Real | M/Z | 0 – 150 | Scientific value of MASS at TIME. This value captures the current measured mass value. |
| 6 | COUNTS_PER_SECOND | Real | Hz | N/A | Scientific value of COUNTS_PER_SECOND at TIME. This value captures the number of counts per second detected with the active multiplier during the duration of the integration period. |
| 7 | DAC_ID | Integer | N/A | N/A | Engineering value of DAC_ID at TIME. This value captures the ID of DAC used during electrode voltage scan (See Table A-2). |
| 8 | DAC_VOLTAGE | Real | V | N/A | Scientific value of DAC_VOLTAGE at TIME. This value captures the voltage setting of the DAC_ID electrode during its voltage scan. |

# Time-corrected message log

The time-corrected message log is an ASCII file identical to the raw message log but with time-corrected stamps (in seconds).

# Species abundance table

This table contains the derived abundances for the primary species measured by the NMS instrument. Abundances will be only reported for species that exhibit a sufficient SNR for the data set.

Table A-7: Definition of species abundance

| # | Name | Format | Units | Range | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | TIME. | Real | s | N/A | SCLK timestamp of any corresponding observed value. |
| 2 | SC_ALTITUDE | Real | Km | N/A | Spacecraft altitude at TIME |
| 3 | LOCAL_TIME | Real | Deg | 0 – 360 | Solar Zenith Angle at the spacecraft location |
| 4 | MASS_002 | Real | /cc | N/A | Abundance of species with mass per charge = 2 (Possibly reflected solar wind H, combined on QMS surfaces). |
| 5 | MASS_004 | Real | /cc | N/A | Abundance of species with mass per charge = 4 (Helium). |
| 6 | MASS_012 | Real | /cc | N/A | Abundance of species with mass per charge = 12 (fragment of CH4, CO, CO2). |
| 7 | MASS_015 | Real | /cc | N/A | Abundance of species with mass per charge = 15 (CH3 fragment of CH4). |
| 8 | MASS_016 | Real | /cc | N/A | Abundance of species with mass per charge = 16 (CH4 and O fragment). |
| 9 | MASS_017 | Real | /cc | N/A | Abundance of species with mass per charge = 17 (OH fragment of H2O). |
| 10 | MASS_018 | Real | /cc | N/A | Abundance of species with mass per charge = 18 (H2O). |
| 11 | MASS_020 | Real | /cc | N/A | Abundance of species with mass per charge = 20 (Ne and Ar-402+). |
| 12 | MASS_028 | Real | /cc | N/A | Abundance of species with mass per charge = 28 (N2 and CO) |
| 13 | MASS_032 | Real | /cc | N/A | Abundance of species with mass per charge = 32 (O2 possibly reflected solar wind O, combined on QMS surfaces). |
| 14 | MASS_036 | Real | /cc | N/A | Abundance of species with mass per charge = 36 (Ar-36). |
| 15 | MASS_040 | Real | /cc | N/A | Abundance of species with mass per charge = 40 (Ar-40). |
| 16 | MASS_044 | Real | /cc | N/A | Abundance of species with mass per charge = 44 (CO2). |